Bgp_as path

R1 configure==========================================================

enable

configure terminal

interface s1/0

ip address 192.168.1.1 255.255.255.0

no shutdown

exit

interface s1/1

ip address 172.24.1.1 255.255.255.0

no shutdown

exit

interface loopback0

ip address 10.1.1.1 255.255.255.0

exit

router bgp 100

neighbor 192.168.1.2 remote-as 200

neighbor 172.24.1.3 remote-as 300

network 10.1.1.0 mask 255.255.255.0

exit

R2 ==============================================================

enable

configure terminal

interface s1/0

ip address 192.168.1.2 255.255.255.0

no shutdown

exit

interface loopback0

ip address 10.2.2.2 255.255.255.0

exit

router bgp 200

neighbor 192.168.1.1 remote-as 100

network 10.2.2.0 mask 255.255.255.0

exit

R3 configure=======================================================

enable

configure terminal

interface s1/1

ip address 172.24.1.3 255.255.255.0

no shutdown

exit

interface loopback0

ip address 10.3.3.3 255.255.255.0

exit

router bgp 300

neighbor 172.24.1.1 remote-as 100

network 10.3.3.0 mask 255.255.255.0

exit

## add the exit and  come to root like R3# then run =show ip interface brief on all routers to see list

then =show ip bgp summary  = to show bgp summary .

also =

R2(config)#do sh ip rout on alla rounters

from r2 =do ping 10.3.3.3 source lo0

from r3=do ping 10.2.2.2 source lo0