Ibgp and ebgp

R1 Configuration (AS 65100)=================================

enable

configure terminal

interface fastethernet0/1

ip address 192.168.1.1 255.255.255.0

no shutdown

exit

interface serial1/0

ip address 172.16.1.1 255.255.255.0

no shutdown

exit

interface serial1/1

ip address 172.16.5.1 255.255.255.0

no shutdown

exit

router bgp 65100

network 192.168.1.0

network 172.16.1.0 mask 255.255.255.0

network 172.16.5.0 mask 255.255.255.0

neighbor 172.16.1.2 remote-as 65200

neighbor 172.16.5.3 remote-as 65200

exit

R2 Configuration (AS 65200)=======================================

enable

configure terminal

interface fastethernet0/0

ip address 10.10.10.2 255.255.255.0

no shutdown

exit

interface fastethernet0/1

ip address 192.168.2.2 255.255.255.0

no shutdown

exit

interface serial1/0

ip address 172.16.1.2 255.255.255.0

no shutdown

exit

router ospf 1

network 10.10.10.0 0.0.0.255 area 0

network 192.168.2.0 0.0.0.255 area 1

exit

router bgp 65200

redistribute ospf 1

network 172.16.1.0 mask 255.255.255.0

neighbor 172.16.1.1 remote-as 65100

neighbor 10.10.10.3 remote-as 65200

exit

R3 Configuration (AS 65200)=================================

enable

configure terminal

interface fastethernet0/0

ip address 10.10.10.3 255.255.255.0

no shutdown

exit

interface fastethernet0/1

ip address 192.168.3.3 255.255.255.0

no shutdown

exit

interface serial1/1

ip address 172.16.5.3 255.255.255.0

no shutdown

exit

router ospf 1

network 10.10.10.0 0.0.0.255 area 0

network 192.168.3.0 0.0.0.255 area 2

exit

router bgp 65200

redistribute ospf 1

network 172.16.5.0 mask 255.255.255.0

neighbor 172.16.5.1 remote-as 65100

neighbor 10.10.10.2 remote-as 65200

exit

then run  exit on all routers and  run = show ip interface brief

show ip ospf neighbor

show ip bgp summary

show ip route