MP LS environment

R1  config============================================================================

hostname R1

interface lo0

ip address 1.1.1.1 255.255.255.255

interface f0/0

ip address 10.0.12.1 255.255.255.252

no shutdown

router ospf 1

network 10.0.12.0 0.0.0.3 area 0

network 1.1.1.1 0.0.0.0 area 0

MPLS + LDP (R1)================

mpls label protocol ldp

mpls ip

interface f0/0

mpls ip

=== u should in conf t  that is config

router bgp 65000

neighbor 1.1.1.1 remote-as 65000

neighbor 1.1.1.1 update-source lo0

address-family vpnv4

neighbor 1.1.1.1 activate

neighbor 1.1.1.1 send-community extended

exit-address-family

R2 CONFIG ===================================================================

hostname R2

interface lo0

ip address 2.2.2.2 255.255.255.255

interface f0/0

ip address 10.0.12.2 255.255.255.252

no shutdown

interface f0/1

ip address 10.0.23.1 255.255.255.252

no shutdown

router ospf 1

network 10.0.12.0 0.0.0.3 area 0

network 10.0.23.0 0.0.0.3 area 0

network 2.2.2.2 0.0.0.0 area 0

mpls label protocol ldp

mpls ip

interface f0/0

mpls ip

interface f0/1

mpls ip

R3 config ==============================================

hostname R3

interface lo0

ip address 3.3.3.3 255.255.255.255

interface f0/0

ip address 10.0.23.2 255.255.255.252

no shutdown

router ospf 1

network 10.0.23.0 0.0.0.3 area 0

network 3.3.3.3 0.0.0.0 area 0

mpls label protocol ldp

mpls ip

interface f0/0

mpls ip

u shoukd in  config (conf t)

router bgp 65000

neighbor 1.1.1.1 remote-as 65000

neighbor 1.1.1.1 update-source lo0

address-family vpnv4

neighbor 1.1.1.1 activate

neighbor 1.1.1.1 send-community extended

exit-address-family

=========================================================================

testing

come to root like r2

CHECK (LDP)

show mpls interfaces

show mpls ldp neighbor

from R1

ping 3.3.3.3 source 1.1.1.1

=====================================

add r4  to the r1  with  r1 = f0/1 n r4 f0/0

add r4 to the r3 with r3= f0/1 n r4 f0/0

VRF CONFIG (ON R1 + R3) :

R1 (PE router)

ip vrf CUST-A

rd 65000:1

route-target export 65000:1

route-target import 65000:1

interface f0/1

ip vrf forwarding CUST-A

ip address 192.168.10.1 255.255.255.0

no shutdown

R3 (PE router)

ip vrf CUST-A

rd 65000:1

route-target export 65000:1

route-target import 65000:1

interface f0/1

ip vrf forwarding CUST-A

ip address 192.168.20.1 255.255.255.0

no shutdown

🟡 STEP 3 — CE ROUTER CONFIG===============================================

R4 (Customer router left)==============

interface f0/0

ip address 192.168.10.2 255.255.255.0

no shutdown

ip route 0.0.0.0 0.0.0.0 192.168.10.1

R5 (Customer router right)=========================================

interface f0/0

ip address 192.168.20.2 255.255.255.0

no shutdown

ip route 0.0.0.0 0.0.0.0 192.168.20.1

🔵 STEP 4 — BGP VRF ACTIVATION (IMPORTANT)

On R1==============================================

router bgp 65000

address-family ipv4 vrf CUST-A

redistribute connected

exit-address-family

On R3===================================================

router bgp 65000

address-family ipv4 vrf CUST-A

redistribute connected

exit-address-family

testing =======================

From R4:

ping 192.168.20.2

From R5: ping 192.168.10.2