inter vlan routing.

In configure  router - use  nm-16esw

S0 Complete Configuration############################################

enable

configure terminal

hostname s0

enable secret class

no ip domain-lookup

ip default-gateway 172.17.99.1

line console 0

password cisco

login

exit

line vty 0 15

password cisco

login

exit

vtp mode server

vtp domain vsit

vtp password cisco

interface range fa0/1-24

shutdown

exit

interface range gi0/1-2

shutdown

exit

interface range fa0/1-3

switchport mode trunk

switchport trunk native vlan 99

no shutdown

exit

interface range fa0/5-6

switchport mode trunk

switchport trunk native vlan 99

no shutdown

exit

vlan 99

name management

vlan 10

name staff

vlan 20

name students

vlan 30

name guests

interface vlan 99

ip address 172.17.99.11 255.255.255.0

no shutdown

exit

end

write memory

S1 Complete Configuration###########################################################

enable

configure terminal

hostname s1

enable secret class

no ip domain-lookup

ip default-gateway 172.17.99.1

line console 0

password cisco

login

exit

line vty 0 15

password cisco

login

exit

vtp mode client

vtp domain vsit

vtp password cisco

interface range fa0/1-24

shutdown

exit

interface range gi0/1-2

shutdown

exit

interface range fa0/1-4

switchport mode trunk

switchport trunk native vlan 99

no shutdown

exit

interface vlan 99

ip address 172.17.99.13 255.255.255.0

no shutdown

exit

end

write memory

S2 Complete Configuration################################################################

enable

configure terminal

hostname s2

enable secret class

no ip domain-lookup

ip default-gateway 172.17.99.1

line console 0

password cisco

login

exit

line vty 0 15

password cisco

login

exit

vtp mode client

vtp domain vsit

vtp password cisco

interface range fa0/1-24

shutdown

exit

interface range gi0/1-2

shutdown

exit

interface range fa0/3-6

switchport mode trunk

switchport trunk native vlan 99

no shutdown

exit

interface fa0/11

switchport mode access

switchport access vlan 10

no shutdown

exit

interface fa0/12

switchport mode access

switchport access vlan 20

no shutdown

exit

interface fa0/13

switchport mode access

switchport access vlan 30

no shutdown

exit

interface vlan 99

ip address 172.17.99.12 255.255.255.0

no shutdown

exit

end

write memory

R1 Complete Configuration==================================================================

enable

configure terminal

hostname r1

enable secret class

no ip domain-lookup

line console 0

password cisco

login

exit

line vty 0 15

password cisco

login

exit

interface fastethernet0/1

no shutdown

exit

interface fastethernet0/1.1

encapsulation dot1Q 1

ip address 172.17.1.1 255.255.255.0

exit

interface fastethernet0/1.10

encapsulation dot1Q 10

ip address 172.17.10.1 255.255.255.0

exit

interface fastethernet0/1.20

encapsulation dot1Q 20

ip address 172.17.20.1 255.255.255.0

exit

interface fastethernet0/1.30

encapsulation dot1Q 30

ip address 172.17.30.1 255.255.255.0

exit

interface fastethernet0/1.99

encapsulation dot1Q 99 native

ip address 172.17.99.1 255.255.255.0

exit

end

write memory

PC0 Configuration===================================================

IP Address: 172.17.10.21

Subnet Mask: 255.255.255.0

Default Gateway: 172.17.10.1

PC1 Configuration=================================================================

IP Address: 172.17.20.22

Subnet Mask: 255.255.255.0

Default Gateway: 172.17.20.1

PC2 Configuration=====================================================================

IP Address: 172.17.30.23

Subnet Mask: 255.255.255.0

Default Gateway: 172.17.30.1

Server Configuration=========================================================

IP Address: 172.17.50.254

Subnet Mask: 255.255.255.0

Default Gateway: 172.17.50.1

Verification cmd ===============================================================

show vlan brief

show interfaces trunk

show vtp status

show ip interface brief

Connectivity Test===================================================================

From PC0:

ping 172.17.20.22

ping 172.17.30.23