configure R1========================================================

enable

conf t

hostname R1

interface loopback0

ip address 192.168.1.1 255.255.255.0

exit

interface serial1/0

ip address 209.165.201.1 255.255.255.0

no shutdown

exit

interface serial1/1

ip address 209.165.202.1 255.255.255.0

no shutdown

exit

ip sla 1

icmp-echo 209.165.201.2

frequency 10

exit

ip sla schedule 1 life forever start-time now

track 1 rtr 1 reachability

ip route 0.0.0.0 0.0.0.0 209.165.201.2 track 1

ip route 0.0.0.0 0.0.0.0 209.165.202.3 10

end

write memory

configure R2========================================================

enable

conf t

hostname R2

interface serial1/0

ip address 209.165.201.2 255.255.255.0

no shutdown

exit

interface serial1/2

ip address 209.165.200.2 255.255.255.0

no shutdown

exit

router eigrp 1

network 209.165.201.0 0.0.0.255

network 209.165.200.0 0.0.0.255

no auto-summary

exit

ip route 192.168.1.0 255.255.255.0 209.165.201.1

ip sla responder

end

write memory

ROUTER 3 configure=====================================================================

enable

conf t

hostname R3

interface serial1/1

ip address 209.165.202.3 255.255.255.0

no shutdown

exit

interface serial1/2

ip address 209.165.200.3 255.255.255.0

no shutdown

exit

router eigrp 1

network 209.165.202.0 0.0.0.255

network 209.165.200.0 0.0.0.255

no auto-summary

exit

ip route 192.168.1.0 255.255.255.0 209.165.202.1

end

write memory

in short :

####R1 → R2 → network

If R2 fails:

R1 → R3 → network  ####

on all routers -show ip interface brief

show ip route

on R1= show track

show ip sla statistics