R1 config=======================================================================

enable

configure terminal

hostname r1

interface serial1/0

ip address 172.16.12.1 255.255.255.0

bandwidth 128

no shutdown

exit

interface serial1/1

ip address 172.16.13.1 255.255.255.0

bandwidth 64

no shutdown

exit

interface loopback0

ip address 192.168.1.1 255.255.255.0

exit

router eigrp 1

network 172.16.12.0 0.0.0.255

network 172.16.13.0 0.0.0.255

network 192.168.1.0

no auto-summary

end

write memory

===========================================================================================

R2 config

enable

configure terminal

hostname r2

interface serial1/0

ip address 172.16.12.2 255.255.255.0

bandwidth 128

no shutdown

exit

interface serial1/2

ip address 172.16.23.2 255.255.255.0

bandwidth 128

no shutdown

exit

interface loopback0

ip address 192.168.2.2 255.255.255.0

exit

router eigrp 1

network 172.16.12.0 0.0.0.255

network 172.16.23.0 0.0.0.255

network 192.168.2.0

no auto-summary

end

write memory

R3 config ======================================================================================

enable

configure terminal

hostname r3

interface serial1/1

ip address 172.16.13.3 255.255.255.0

bandwidth 64

no shutdown

exit

interface serial1/2

ip address 172.16.23.3 255.255.255.0

bandwidth 128

no shutdown

exit

interface serial1/3

ip address 172.16.34.3 255.255.255.0

bandwidth 64

no shutdown

exit

interface loopback0

ip address 192.168.3.3 255.255.255.0

exit

router eigrp 1

network 172.16.13.0 0.0.0.255

network 172.16.23.0 0.0.0.255

network 172.16.34.0 0.0.0.255

network 192.168.3.0

no auto-summary

PBR Configuration on R3====================

ip access-list standard pbr-acl

permit 192.168.5.0 0.0.0.255

exit

route-map r3-to-r1 permit 10

match ip address pbr-acl

set ip next-hop 172.16.13.1

exit

interface serial1/3

ip policy route-map r3-to-r1

exit

end

write memory

R4 config================================================================================

enable

configure terminal

hostname r4

interface serial1/3

ip address 172.16.34.4 255.255.255.0

bandwidth 64

no shutdown

exit

interface loopback0

ip address 192.168.4.1 255.255.255.0

exit

interface loopback1

ip address 192.168.5.1 255.255.255.0

exit

router eigrp 1

network 172.16.34.0 0.0.0.255

network 192.168.4.0

network 192.168.5.0

no auto-summary

end

write memory

===========================================================================================

run on all  routers ===

show ip interface brief

show ip eigrp neighbors

show ip route

from R4 ==

ping 192.168.1.1

Traceroute Before PBR

On R4:

traceroute 192.168.1.1 source 192.168.4.1

traceroute 192.168.1.1 source 192.168.5.1

both will uses  r4-r3-r3-r1

after pbr ===

traceroute 192.168.1.1 source 192.168.4.1

r4-r3-r2-r1   cz  its not  set  to use pbr

traceroute 192.168.1.1 source 192.168.5.1

it will r4 -r3-r1   we  configure  to choose the hope