R1 Configuration================================================================================

enable

configure terminal

hostname r1

interface serial1/0

ip address 10.1.1.1 255.255.255.0

no shutdown

exit

interface loopback0

ip address 192.168.1.1 255.255.255.0

exit

ip route 0.0.0.0 0.0.0.0 10.1.1.2

security password min-length 10

enable secret class12345

line console 0

password ciscoconpass

login

exec-timeout 5 0

logging synchronous

exit

line vty 0 4

password ciscovtypass

login

exec-timeout 5 0

exit

line aux 0

no exec

exit

service password-encryption

banner motd $Unauthorized access not allowed$

end

write memory

R2 Configuration ========================================================================

enable

configure terminal

interface serial1/0

ip address 10.1.1.2 255.255.255.0

no shutdown

exit

interface serial1/1

ip address 10.2.2.2 255.255.255.0

no shutdown

exit

ip route 192.168.1.0 255.255.255.0 10.1.1.1

ip route 192.168.3.0 255.255.255.0 10.2.2.3

end

write memory

R3 Configuration===============================================================

enable

configure terminal

hostname r3

interface serial1/1

ip address 10.2.2.3 255.255.255.0

no shutdown

exit

interface loopback0

ip address 192.168.3.3 255.255.255.0

exit

ip route 0.0.0.0 0.0.0.0 10.2.2.2

security password min-length 10

enable secret class12345

line console 0

password ciscoconpass

login

exec-timeout 5 0

logging synchronous

exit

line vty 0 4

password ciscovtypass

login

exec-timeout 5 0

exit

line aux 0

no exec

exit

service password-encryption

banner motd $Unauthorized access not allowed$

end

write memory

===========================================================================

verification on R1

show ip interface brief

show ip route

ping 192.168.3.3

on R3 ==

show ip interface brief

show ip route

ping 192.168.1.1

telnet 10.1.1.1

paswsord=ciscovtypass(just enter pass)

enable

show controllers serial 1/0