R1===========================================================================

conf t

hostname R1

ip cef

mpls label protocol ldp

ip vrf RED

rd 100:1

route-target both 100:1

interface Loopback0

ip address 1.1.1.1 255.255.255.255

interface FastEthernet0/0

ip address 10.0.0.1 255.255.255.0

mpls ip

no shutdown

interface FastEthernet0/1

ip vrf forwarding RED

ip address 192.168.1.1 255.255.255.0

no shutdown

router ospf 1

router-id 1.1.1.1

network 1.1.1.1 0.0.0.0 area 0

network 10.0.0.0 0.0.0.255 area 0

router ospf 2 vrf RED

router-id 1.1.1.1

network 192.168.1.0 0.0.0.255 area 0

redistribute bgp 100 subnets

router bgp 100

bgp log-neighbor-changes

neighbor 3.3.3.3 remote-as 100

neighbor 3.3.3.3 update-source Loopback0

address-family vpnv4

neighbor 3.3.3.3 activate

neighbor 3.3.3.3 send-community both

exit-address-family

address-family ipv4 vrf RED

redistribute ospf 2

exit-address-family

end

wr

R2 Configuration=========================================================================

conf t

hostname R2

ip cef

mpls ip

mpls label protocol ldp

interface Loopback0

ip address 2.2.2.2 255.255.255.255

interface FastEthernet0/0

ip address 10.0.0.2 255.255.255.0

mpls ip

no shutdown

interface FastEthernet0/1

ip address 10.0.1.2 255.255.255.0

mpls ip

no shutdown

router ospf 1

router-id 2.2.2.2

network 2.2.2.2 0.0.0.0 area 0

network 10.0.0.0 0.0.0.255 area 0

network 10.0.1.0 0.0.0.255 area 0

R3 Configuration========================================================================conf t

hostname R3

ip cef

mpls label protocol ldp

ip vrf RED

rd 100:1

route-target both 100:1

interface Loopback0

ip address 3.3.3.3 255.255.255.255

interface FastEthernet0/0

ip address 10.0.1.3 255.255.255.0

mpls ip

no shutdown

interface FastEthernet0/1

ip vrf forwarding RED

ip address 192.168.2.3 255.255.255.0

no shutdown

router ospf 1

router-id 3.3.3.3

network 3.3.3.3 0.0.0.0 area 0

network 10.0.1.0 0.0.0.255 area 0

router ospf 2 vrf RED

router-id 3.3.3.3

network 192.168.2.0 0.0.0.255 area 0

redistribute bgp 100 subnets

router bgp 100

bgp log-neighbor-changes

neighbor 1.1.1.1 remote-as 100

neighbor 1.1.1.1 update-source Loopback0

address-family vpnv4

neighbor 1.1.1.1 activate

neighbor 1.1.1.1 send-community both

exit-address-family

address-family ipv4 vrf RED

redistribute ospf 2

exit-address-family

end

wr

R4 Configuration=================================================================

conf t

hostname R4

interface Loopback0

ip address 4.4.4.4 255.255.255.255

interface FastEthernet0/0

ip address 192.168.1.4 255.255.255.0

no shutdown

router ospf 2

router-id 4.4.4.4

network 192.168.1.0 0.0.0.255 area 0

network 4.4.4.4 0.0.0.0 area 0

R5 Configuration=============================================================

conf t

hostname R5

interface Loopback0

ip address 6.6.6.6 255.255.255.255

interface FastEthernet0/0

ip address 192.168.2.6 255.255.255.0

no shutdown

router ospf 2

router-id 6.6.6.6

network 192.168.2.0 0.0.0.255 area 0

network 6.6.6.6 0.0.0.0 area 0

===========================================================================

Testing

r1 . r2 .r3

r1#show ip ospf neighbor =Verify OSPF Neighbors

r1#show mpls interfaces =Verify MPLS Interfaces

r1#show mpls ldp neighbor =verify LDP neighbours

show ip route = verify core ruoting table

show ip route vrf RED  vrf routing table

on r1 n r3

show bgp vpnv4 unicast all summary

show ip bgp vpnv4 vrf RED

from r4

ping 6.6.6.6

traceroute 6.6.6.6

from r5

ping 4.4.4.4