CONTRIBUTION

AI Agent Interworking — Implementation Study

An open-source oneM2M–MCP interworking implementation and evaluation, structured along oneM2M TR-0081 "AI Agent Interworking" V0.2.0

| Field | Value |
| --- | --- |
| Document type | Input contribution / implementation study |
| Related oneM2M work | WI-0122; TR-0081 V0.2.0 (Release 6) |
| Source | Rostan Technologies |
| Contact | Rostan Technologies — virender@rostantechnologies.com |
| Date | 2026-08-05 |
| Status | Draft for discussion |

Notice: This document is an independent input contribution. It is NOT an official oneM2M deliverable and does not represent the position of oneM2M or its partners. The clause structure follows TR-0081 V0.2.0 to ease mapping of this material into the ongoing work.

# 1  Scope

The present document reports implementation experience with oneM2M–MCP interworking: an open-source MCP server that exposes a oneM2M CSE to AI agents. It describes the architecture options considered, the security model adopted, a working reference implementation on the ACME CSE, and first performance measurements. It is intended as input to the studies collected in TR-0081 [i.1] under work item WI-0122.

# 2  References

## 2.1  Normative references

[1]  oneM2M TS-0001: "Functional Architecture".

[2]  oneM2M TS-0004: "Service Layer Core Protocol Specification".

[3]  oneM2M TS-0003: "Security Solutions".

[4]  Model Context Protocol specification, https://modelcontextprotocol.io (governed under the Linux Foundation Agentic AI Foundation).

## 2.2  Informative references

[i.1]  oneM2M TR-0081: "AI Agent Interworking", V0.2.0, 2026-06. https://specifications.onem2m.org/tr/tr-0081/latest/

[i.2]  oneM2M WI-0122: "oneM2M and Model Context Protocol". https://www.onem2m.org/technical/published-specifications/release-6

[i.3]  I. Friese: "Early experiments combining Agentic AI, MCP, and oneM2M", oneM2M executive viewpoint, 2025. https://www.onem2m.org/membership/executive-viewpoints/968-ingo-friese-ai-mcp

[i.4]  ACME oneM2M CSE, open-source CSE for education, https://github.com/ankraft/ACME-oneM2M-CSE

[i.5]  oneM2M TS-0033: "Interworking Framework".

[i.6]  I. Friese, MCP_oneM2M_example — single-tool MCP demonstration over an ACME CSE, March 2026. https://github.com/ingofriese/MCP_oneM2M_example

# 3  Definition of terms and abbreviations

## 3.1  Terms

AI agent: software system that uses a large language model to plan and execute actions towards a goal, using external tools.

MCP host: agent application that manages one or more MCP clients (e.g. an AI assistant, an IDE, an orchestration runtime).

MCP server: endpoint that exposes tools, resources and prompts to MCP clients over a defined transport.

tool: callable function exposed by an MCP server, described so that an agent can decide when and how to invoke it.

originator: oneM2M identifier (e.g. AE-ID) on whose behalf a request primitive is executed and against which access control is evaluated.

## 3.2  Abbreviations

| Abbrev. | Meaning |
| --- | --- |
| ACP | Access Control Policy |
| AE | Application Entity |
| CIN | Content Instance |
| CNT | Container |
| CSE | Common Services Entity |
| IPE | Interworking Proxy Entity |
| LLM | Large Language Model |
| MCP | Model Context Protocol |
| RSC | Response Status Code |
| SUB | Subscription |

# 4  Conventions

The key words "should", "may" and "can" in the present document are to be interpreted as described in the oneM2M Drafting Rules. This document contains no normative provisions.

# 5  Overview

## 5.1  Agentic AI

Agentic AI systems combine a language model with planning and tool use: the model decides which tools to call, interprets the results, and iterates until a goal is reached. For IoT, this enables natural-language operation of complex systems ("find all temperature sensors and alert me when any exceeds 30 °C") without bespoke application development. The prerequisite is that the IoT system exposes its capabilities in a form agents understand — which MCP standardizes.

## 5.2  Architecture overview

MCP follows a client-server architecture. An MCP host (the agent application) runs one MCP client per connected server. Servers expose tools with human/machine-readable descriptions; the LLM selects and invokes them. Figure 5.2-1 shows the general architecture with a oneM2M CSE as one of the connected systems.

MCP Host (agent application)

AI agent / LLM

MCP

client 1

MCP

client 2

MCP server A

e.g. database

MCP server B

oneM2M CSE

External

service

oneM2M

resource tree

MCP (JSON-RPC)

MCP (JSON-RPC)

oneM2M primitives

Figure 5.2-1: MCP architecture with a oneM2M CSE as external system

## 5.3  Transport protocol and message format

MCP messages are JSON-RPC 2.0. The implementation described here uses the streamable-HTTP transport (a single HTTP endpoint supporting streaming responses and server-initiated notifications), which suits CSE deployment better than stdio and enables remote agents. On the oneM2M side, all operations are expressed as request primitives per TS-0004 [2] (operation, target, originator, request identifier, release version, primitive content) and executed by the CSE.

## 5.4  Lifecycle

An MCP session proceeds: initialize (capability negotiation) → tool discovery (list_tools) → tool invocation (call_tool, repeated) → termination. Server-initiated notifications can inform the agent about asynchronous events; clause 7 discusses how oneM2M subscriptions can be bridged to such notifications as further work.

# 6  Architecture model of MCP to oneM2M interworking

## 6.1  Design considerations

Reuse the CSE's own request pipeline. Agent actions must pass through the same validation, access-control and dispatch path as any AE request, so no parallel/privileged access path is created.

Curated tools over raw primitives. Agents perform better with a small set of task-oriented tools (discover, read, store, subscribe) than with a single generic "send primitive" tool; a guarded raw tool remains useful for expert use.

Tool descriptions teach oneM2M. Descriptions embed the minimum oneM2M concepts (resource tree, containers, contentInstances) so models use the standard correctly without fine-tuning.

Deployment neutrality. The interworking function should be deployable both inside a CSE (plugin) and as a standalone gateway in front of any CSE (AE/IPE over Mca).

## 6.2  Interworking scenarios

### 6.2.1  Scenario A: MCP server as CSE plugin

The MCP server runs inside the CSE process and injects primitives directly into the request pipeline (Figure 6.2-1). This minimizes latency and deployment complexity and was chosen for the reference implementation (clause 6.3). It requires a CSE with an extension mechanism, such as the ACME CSE plugin framework [i.4].

AI agent

MCP host + client

oneM2M CSE (e.g. ACME)

MCP server

plugin

Request pipeline

validation · ACP · dispatch

Resource tree

AE / CNT / CIN / SUB

Protocol bindings

HTTP · MQTT · CoAP · WS

MCP

(streamable-HTTP)

primitives

other AEs

Figure 6.2-1: Scenario A — MCP server as a plugin of the CSE

### 6.2.2  Scenario B: standalone MCP gateway (AE/IPE)

The MCP server runs as a separate process, registers as an AE (or is structured as an IPE per TS-0033 [i.5]) and communicates with the CSE over Mca (Figure 6.2-2). This works with any compliant CSE (Mobius, OM2M, tinyIoT) at the cost of one additional network hop, and is the natural evolution path for a CSE-independent deliverable.

AI agent

MCP host + client

MCP gateway

registered as AE / IPE

oneM2M CSE

any implementation

MCP (JSON-RPC)

Mca (HTTP/MQTT)

Figure 6.2-2: Scenario B — standalone MCP gateway over Mca

### 6.2.3  Relation to the agentic IPE generation framework

TR-0081 clause 6.2.1 [i.1] describes a framework in which a coding agent autonomously generates IPEs, interacting with the CSE through a "oneM2M MCP Server" supported by oneM2M-specific skills. The implementation described in the present document provides exactly that component and can therefore serve as the CSE-side building block of the framework; the tool set of clause 6.3 covers the resource preparation steps named there (creation of <AE> and <container> resources, storage and validation of <contentInstance> resources).

## 6.3  Reference implementation

A single-tool MCP demonstration over an ACME CSE was published by the WI-0122 rapporteur in March 2026 [i.6], establishing feasibility. The implementation reported here extends that to a deployable server. A reference implementation of Scenario A exists as a user plugin for the ACME CSE (v2026.05 plugin architecture; no fork or core modification). It exposes seventeen tools; Table 6.3-1 shows the core mapping to oneM2M primitives. Figure 6.3-1 shows the message flow of a tool call.

| MCP tool | oneM2M primitive | Notes |
| --- | --- | --- |
| cse_info | RETRIEVE <CSEBase> | entry point for orientation |
| discover_resources | RETRIEVE + filterCriteria (fu=1, ty, lbl) | returns resource identifier list |
| retrieve_resource | RETRIEVE | structured/unstructured IDs |
| register_ae | CREATE <AE> | agent-owned application entity |
| create_container | CREATE <container> | with retention policy (mni) |
| add_content | CREATE <contentInstance> | data point write |
| get_latest_content | RETRIEVE <container>/la | newest data point |
| create_subscription | CREATE <subscription> | notification URI |
| delete_resource | DELETE | annotated destructive |
| send_onem2m_primitive | any | guarded expert escape hatch |

Table 6.3-1: MCP tool to oneM2M primitive mapping

AI agent

MCP server

plugin

Request

pipeline

Resource tree

call_tool("add_content", {container, value})

CREATE primitive {op:1, to, fr, pc:cin}

validate + ACP check

store <contentInstance>

created

Result (rsc = 2001, resource)

tool result (JSON)

Figure 6.3-1: Message flow of an agent tool call (data point write)

# 7  Security aspects of MCP interworking

## 7.1  Overview

Agentic access raises the security questions identified in [i.3]: how is an agent authenticated, what is it authorized to do, and how are notifications and dependability handled. The guiding principle of this implementation: the agent is a oneM2M security principal like any other — no new authorization model is invented.

## 7.2  Security architecture

Security is layered (Figure 7.2-1): transport security and agent authentication are handled at the MCP layer (TLS; the MCP specification supports OAuth 2.0-based authorization); each authenticated agent is then mapped to a dedicated oneM2M originator (AE-ID); every primitive executes under that originator and is authorized by the CSE's <accessControlPolicy> evaluation per TS-0003 [3].

AI agent

MCP endpoint

transport auth

(TLS, OAuth)

Originator

mapping

agent → AE-ID

<ACP>

check

Every tool call is authorized by the CSE's own <accessControlPolicy> evaluation —

the agent has no more rights than its mapped originator.

Figure 7.2-1: Layered security architecture

## 7.3  Security procedure for MCP interworking

1. The administrator registers a dedicated <AE> for the agent and assigns <accessControlPolicy> resources granting only the required operations on the required subtrees.

2. The MCP endpoint authenticates the agent (transport-level; OAuth token verification where required).

3. Each tool call is mapped to a primitive with the agent's AE-ID as originator; the CSE grants or denies per its ACP evaluation.

4. Denials are returned to the agent as oneM2M response status codes, allowing it to explain and adapt rather than fail silently.

## 7.4  Open issues for further study

Per-session/per-user originator mapping when one MCP endpoint serves multiple agents.

Bridging oneM2M <subscription> notifications into MCP server-initiated notifications (dependability semantics, delivery guarantees).

Rate limiting and congestion protection for agent-generated request storms.

Auditability: correlating MCP session identity with oneM2M request records.

# 8  Evaluation

First measurements on commodity hardware (4-core Xeon, CSE and client co-located; ACME CSE 2026.05.1; 300 requests per cell; logging off):

| Path | Operation | p50 latency | Throughput |
| --- | --- | --- | --- |
| Direct oneM2M HTTP | CIN CREATE | 3.8 ms | 254 req/s |
| Direct oneM2M HTTP | latest RETRIEVE | 2.0 ms | 492 req/s |
| Via MCP (agent path) | CIN CREATE tool | 6.2 ms | ~184 calls/s |
| Via MCP (agent path) | latest RETRIEVE tool | 4.2 ms | (combined) |
| Via MCP, 5 parallel sessions | mixed | 17–19 ms | 242 calls/s |

Table 8-1: Measured overhead of the MCP interworking layer

The MCP layer adds approximately 2 ms per operation over the direct HTTP binding — negligible for agentic workloads. The result indicates that interworking cost is not an obstacle to Scenario A deployments.

# 9  Conclusions and proposed next steps

oneM2M–MCP interworking is practical today: a complete agent-to-CSE path exists with negligible overhead, using only standardized oneM2M mechanisms for authorization.

We propose to contribute the tool-to-primitive mapping (Table 6.3-1) and the layered security procedure (clause 7.3) as input to TR-0081.

Both scenarios are implemented from one codebase and pass the same test suite, so the interworking function is not tied to a single CSE.

Subscription notifications are bridged back to the agent, and semantic query over <semanticDescriptor> is exposed, so an agent can find resources by meaning (SAREF/SPARQL) rather than by tree position.

An agent benchmark over discovery, data, ACP-restricted access, subscriptions and semantics has been built and run: Claude Sonnet 5 scores 6/6. Two non-LLM controls (a correct solver and a do-nothing agent that claims success) bound the harness at 6/6 and 0/6.

Remaining work: benchmark results across further models and providers.

The implementation is available as open source (BSD-3) for use by oneM2M members and the developer community.

# History

| Version | Date | Description |
| --- | --- | --- |
| 0.1.0 | 2026-08-05 | First draft: architecture scenarios, security model, reference implementation on ACME CSE, first measurements. |