Driver https://www.pololu.com/file/0J14/pololu-cp2102-windows-220616.zip

1. ADD Device

Select "ESP32"

Select "ESP32 Dev Module"

press "Continue"

Rename Device example "LED"

2. Create Thing

Rename Thing: "LED"

click "+ Variable"

Select "Boolean"

Variable name: LED

Read & Write

On change

Click "Create

3. Create Dashboard

Rename Dashboard: LED

Click "EDIT"

Click "ADD"

Add "PUSH BUTTON"

Click "Link Variable"

Select variable "LED"

Click "Done"

4. Go TO DEVICES

Open "LED"

Click "CHANGE NETWORK"

Click "CREATE NEW"

Set WIFI Credentials and Secret Key

Click "GO TO SKETCH"

5. Open ARDUINO IDE

BOARDS MANAGER: ESP32    (esp32 by Espressif Systems)

LIBRARY MANAGER: ArduinoIoTCloud    (ArduinoloTCloud by Arduino)

Click "Cloud Sketchbook"

Sign-in Account

PULL the Sketchbook

Click 3 dots and open in new Window

Paste Code

6. Code:

#include "arduino_secrets.h"

#include "thingProperties.h"

#define LED_PIN 2   // ESP32 Built-in LED usually GPIO2

void setup() {

Serial.begin(9600);

delay(1500);

// Initialize LED pin

pinMode(LED_PIN, OUTPUT);

digitalWrite(LED_PIN, LOW);

// Initialize IoT Cloud properties

initProperties();

// Connect to Arduino IoT Cloud

ArduinoCloud.begin(ArduinoIoTPreferredConnection);

setDebugMessageLevel(2);

ArduinoCloud.printDebugInfo();

}

void loop() {

ArduinoCloud.update();

// Write LED based on Cloud variable

digitalWrite(LED_PIN, LED);

}

/*

This function runs whenever iOT1 changes from dashboard

*/

void onLEDChange() {

Serial.print("Cloud LED Value Changed: ");

Serial.println(LED);

digitalWrite(LED_PIN, LED);

}

__________________________________________________

7. Connection

Connect Anode of LED (+) to Pin D2 or Pin GPIO2 of ESP32.

Connect Cathode of LED (-) to Pin GND of ESP32.

Connect ESP32 to CPU

8. Click "File"

Click "Preferences"

Additional boards manager URLs:

https://dl.espressif.com/dl/package_esp32_index.json

Click "OK"

9. Tools → Board and select ESP32 Dev Module

Tools → Port and select the correct COM port (COM3/COM5)

10. Upload the code by clicking "→"