Driver : https://www.pololu.com/file/0J14/pololu-cp2102-windows-220616.zip

1. ADD Device

Select "ESP32"

Select "ESP32 Dev Module"

press "Continue"

Rename Device example "Temp"

2. Create Thing

Rename Thing: Temp

click "+ Variable"

Select "Floating point number"

Variable name: Temp

Read only

On change

Click "Create

3. Create Dashboard

Rename Dashboard: Temp

Click "EDIT"

Click "ADD"

Add "GUAGE"

Click "Link Variable"

Select variable "Temp"

Click "Done"

4. Go TO DEVICES

Open "Temp"

Click "CHANGE NETWORK"

Click "CREATE NEW"

Set WIFI Credentials and Secret Key

Click "GO TO SKETCH"

5. Open ARDUINO IDE

BOARDS MANAGER: ESP32    (esp32 by Espressif Systems)

LIBRARY MANAGER: DHT sensor library   (DHT sensor library by Adafruit)

Click "Cloud Sketchbook"

Sign-in Account

PULL the Sketchbook

Click 3 dots and open in new Window

Paste Code

6. Code:

#include "arduino_secrets.h"

#include "thingProperties.h"

#include <DHT.h>

#define DHTPIN 2        // GPIO2 (Change if needed)

#define DHTTYPE DHT11   // DHT11 Sensor

DHT dht(DHTPIN, DHTTYPE);

// Forward declaration (important for IoT Cloud)

void onTempChange();

void setup() {

Serial.begin(9600);

delay(1500);

// IoT Cloud init

initProperties();

ArduinoCloud.begin(ArduinoIoTPreferredConnection);

setDebugMessageLevel(2);

ArduinoCloud.printDebugInfo();

// Start DHT sensor

Serial.println("DHT11 Sensor Test...");

dht.begin();

}

void loop() {

ArduinoCloud.update();

static unsigned long lastRead = 0;

if (millis() - lastRead >= 2000) {

lastRead = millis();

// Read temperature

Temp = dht.readTemperature();

// Check if reading failed

if (isnan(Temp)) {

Serial.println("Failed to read from DHT sensor!");

return;

}

// Print temperature

Serial.print("Temperature: ");

Serial.print(Temp);

Serial.println(" °C");

}

}

/*

Runs if value is changed from dashboard (optional)

*/

void onTempChange() {

// Usually not needed for sensor (cloud → device)

}

__________________________________________________

7. Connect sensor DATA Pin to Pin D2 of ESP32.

Connect GND (-) to Pin GND of ESP32.

Connect VCC (+) to Pin 3V3 of ESP32.

Connect ESP32 to CPU

8. Click "File"

Click "Preferences"

Additional boards manager URLs:

https://dl.espressif.com/dl/package_esp32_index.json

Click "OK"

9. Tools → Board and select ESP32 Dev Module

Tools → Port and select the correct COM port (COM3/COM5)

10. Upload the code by clicking "→"

11. Tools → Serial Monitor