Driver : https://www.pololu.com/file/0J14/pololu-cp2102-windows-220616.zip

1. ADD Device

Select "ESP32"

Select "ESP32 Dev Module"

press "Continue"

Rename Device example "Fire"

2. Create Thing

Rename Thing: Fire

click "+ Variable"

Select "Boolean"

Variable name: fire

Read & write

On change

Click "Create

3. Create Dashboard

Rename Dashboard: Fire

Click "EDIT"

Click "ADD"

Add "LED"

Click "Link Variable"

Select variable "fire"

Select "Double LEDs"

Set  True → RED

Set  False → GREEN

Click "Done"

4. Go TO DEVICES

Open "Fire"

Click "CHANGE NETWORK"

Click "CREATE NEW"

Set WIFI Credentials and Secret Key

Click "GO TO SKETCH"

5. Open ARDUINO IDE

BOARDS MANAGER: ESP32    (esp32 by Espressif Systems)

LIBRARY MANAGER: ArduinoIoTCloud    (ArduinoloTCloud by Arduino)

Click "Cloud Sketchbook"

Sign-in Account

PULL the Sketchbook

Click 3 dots and open in new Window

Paste Code

6. Code:

#include "arduino_secrets.h"

#include "thingProperties.h"

#define FLAME_PIN 4

void setup() {

Serial.begin(9600);

delay(1500);

pinMode(FLAME_PIN, INPUT);

initProperties();

ArduinoCloud.begin(ArduinoIoTPreferredConnection);

setDebugMessageLevel(2);

ArduinoCloud.printDebugInfo();

Serial.println("Flame sensor started!");

}

void loop() {

ArduinoCloud.update();

int flame = digitalRead(FLAME_PIN);

// Assuming LOW means flame detected

fire = (flame == LOW);

if (fire) {

Serial.println("Flame detected!");

} else {

Serial.println("No flame");

}

delay(2000);

}

void onFireChange() {

// Add your code here to act upon fire change (optional)

}

__________________________________________________

7. Connect sensor DATA Pin to Pin D4 (GPIO4) of ESP32.

Connect GND (-) to Pin GND of ESP32.

Connect VCC (+) to Pin 3V3 of ESP32.

8. Click "File"

Click "Preferences"

Additional boards manager URLs:

https://dl.espressif.com/dl/package_esp32_index.json

Click "OK"

9. Tools → Board and select ESP32 Dev Module

Tools → Port and select the correct COM port (COM3/COM5)

10. Upload the code by clicking "→"

11. Tools → Serial Monitor