Driver: https://www.pololu.com/file/0J14/pololu-cp2102-windows-220616.zip

1. ADD Device

Select "ESP32"

Select "ESP32 Dev Module"

press "Continue"

Rename Device example "Motion"

2. Create Thing

Rename Thing: Motion

click "+ Variable"

Select "Boolean"

Variable name: Motion

Read & write

On change

Click "Create

3. Create Dashboard

Rename Dashboard: Motion

Click "EDIT"

Click "ADD"

Add "STATUS"

Click "Link Variable"

Select variable "Motion"

Select  Status Labels → Icons

Click "Done"

4. Go TO DEVICES

Open "Motion"

Click "CHANGE NETWORK"

Click "CREATE NEW"

Set WIFI Credentials and Secret Key

Click "GO TO SKETCH"

5. Open ARDUINO IDE

BOARDS MANAGER: ESP32    (esp32 by Espressif Systems)

LIBRARY MANAGER: ArduinoIoTCloud    (ArduinoloTCloud by Arduino)

Click "Cloud Sketchbook"

Sign-in Account

PULL the Sketchbook

Click 3 dots and open in new Window

Paste Code

6. Code:

#include "arduino_secrets.h"

#include "thingProperties.h"

#define PIR_PIN 4

#define LED_PIN 2

// Forward declaration

void onMotionChange();

// Track previous state

bool lastMotionState = false;

void setup() {

Serial.begin(9600);

delay(1500);

pinMode(PIR_PIN, INPUT);

pinMode(LED_PIN, OUTPUT);

initProperties();

ArduinoCloud.begin(ArduinoIoTPreferredConnection);

setDebugMessageLevel(2);

ArduinoCloud.printDebugInfo();

Serial.println("PIR Motion Sensor Started...");

}

void loop() {

ArduinoCloud.update();

// Read PIR sensor

bool currentState = digitalRead(PIR_PIN);

// Only act if state changes

if (currentState != lastMotionState) {

lastMotionState = currentState;

// Update cloud variable

Motion = currentState;

// Control LED

digitalWrite(LED_PIN, currentState ? HIGH : LOW);

if (currentState) {

Serial.println("Motion detected!");

} else {

Serial.println("No motion");

}

}

}

/*

Runs if value is changed from dashboard (optional)

*/

void onMotionChange() {

// Optional: you could control LED from cloud here if needed

}

__________________________________________________

7. Connect sensor DATA Pin to Pin D4 (GPIO4) of ESP32.

Connect GND (-) to Pin GND of ESP32.

Connect VCC (+) to Pin 3V3 of ESP32.

8. Click "File"

Click "Preferences"

Additional boards manager URLs:

https://dl.espressif.com/dl/package_esp32_index.json

Click "OK"

9. Tools → Board and select ESP32 Dev Module

Tools → Port and select the correct COM port (COM3/COM5)

10. Upload the code by clicking "→"

11. Tools → Serial Monitor