## Step 17 Filter tests

Practical Recommendation

A sensible validation sequence before trusting the tuned-sensor results:

- Run a simple two-vehicle scenario (lead + one CACC follower) with sensor_model='passthrough' and record the ground-truth trajectories.

- Switch to sensor_model='kalman' and overlay the filtered estimates on the ground truth. The gap estimate should track within the noise band; gap_rate should track without excessive lag.

- Switch to comms_model='dsrc' and check that lead_a_radar converges to a reasonable estimate of the leader's true acceleration within a few seconds of simulation start.

- Only then run multi-vehicle string stability scenarios, because any filter mistuning will manifest differently at each platoon position and could be mistaken for a genuine string stability result.

The architecture makes this validation sequence straightforward — the passthrough baseline is always available as a reference by changing one Excel field.

You said: What should the driving profile be for the lead vehicle?

What should the driving profile be for the lead vehicle?

Claude responded: This depends on what you're trying to learn from the simulation, and different profiles are appropriate for different validation stages.

Examined validation requirements and filter-tuning considerations

Examined validation requirements and filter-tuning considerations

This depends on what you're trying to learn from the simulation, and different profiles are appropriate for different validation stages. The right answer is actually a sequence of profiles of increasing complexity, each revealing something the previous one cannot.

Stage 1 — Constant Speed (Validation Baseline)

time_s    command      value    transition

0         set_speed    30.0     step

Run this first, before anything else. At constant speed the true gap between any two vehicles should be constant after the initial transient, true relative velocity should be zero, and true leader acceleration should be zero. This means:

- The KF's gap estimate should converge to the true gap and hold there

- gap_rate should converge to zero and stay there

- lead_a_radar from the fusion KF should converge to zero

Any deviation from these expected values is a filter bug or tuning problem, not physics. This profile makes filter validation unambiguous precisely because the ground truth is trivially known. It is also the profile that exposes the initialization transient discussed in the previous answer — you can see exactly how long convergence takes.

Stage 2 — Single Step Change (Impulse Response)

time_s    command      value    transition

0         set_speed    30.0     step

60        set_speed    25.0     step

120       set_speed    30.0     step

A step change in speed produces a sudden acceleration pulse in the lead vehicle — the controller responds to the error between current speed and target speed proportionally, so kp * (25.0 - 30.0) gives an immediate deceleration command. This is the hardest input for the CACC chain to handle cleanly because it has the most high-frequency content.

What this reveals:

- Whether the gap KF tracks a rapid change without excessive lag

- Whether lead_a_radar from the fusion KF detects the deceleration quickly enough to be useful feedforward

- Whether the CACC controller maintains string stability — follower amplitudes should not exceed lead amplitude

- The BSM's role: a fresh BSM packet arriving during the deceleration event should visibly improve the follower's response compared to the passthrough case

The step transition is deliberately unrealistic — real vehicles can't change speed instantaneously — but that's exactly what makes it useful for validation. It stress-tests the filters with the sharpest possible input.

Stage 3 — Ramp (Realistic Acceleration Event)

time_s    command      value    transition

0         set_speed    30.0     step

60        set_speed    20.0     ramp

90        set_speed    30.0     ramp

150       set_speed    30.0     step

The ramp transition linearly interpolates between speeds, producing a roughly constant acceleration during the transition — more physically realistic than the step because real vehicles are limited by actuator dynamics. The 30-second ramp duration at 10 m/s speed change gives approximately -0.33 m/s² average deceleration, which is a gentle but noticeable event well within comfortable driving range.

This profile is better than the step for studying the CACC controller's steady-state behavior during following because the filter has time to track the changing acceleration rather than being hit with a discontinuity. The difference between passthrough and realistic sensor results should be visible but not dramatic — which is the right regime for a fair comparison.

Stage 4 — Sinusoidal (String Stability Test)

sine_amplitude_mps:   2.0

sine_frequency_hz:    0.1

sine_offset_mps:      30.0

sine_warmup_s:        60.0

This is the profile described in Section 18 of the design notes, reserved for after the filter validation stages are complete. A 0.1 Hz sinusoid with 2 m/s amplitude around 30 m/s cruise gives smooth, continuous, predictable oscillation. The 60-second warmup ensures all transients have settled before the oscillation begins so the amplitude measurements are clean.

What this uniquely reveals that no other profile can:

- Empirical string stability — whether follower acceleration amplitudes grow or shrink down the platoon

- The direct comparison between passthrough and realistic sensors in terms of string stability margin — does sensor noise erode it?

- Whether the fusion KF's lead_a_radar estimate improves or degrades string stability relative to using BSM feedforward alone

The frequency of 0.1 Hz is chosen deliberately. Below about 0.05 Hz the platoon has time to fully settle between oscillation peaks and the result approaches quasi-static. Above about 0.3 Hz you're approaching the bandwidth limits of the actuator response model, and the results become sensitive to the lag model parameters rather than the control law. The 0.1 Hz sweet spot produces clear oscillation that the controllers must actively manage.

Stage 5 — Emergency Braking (Safety Test)

time_s    command      value    transition

0         set_speed    30.0     step

120       set_speed    0.0      step

A commanded stop from highway speed. This is not primarily a filter validation scenario — it's a safety scenario. The step to zero will saturate the lead vehicle's deceleration at a_min since the proportional controller clips at its limits. You're looking for whether any following vehicle's gap goes negative (collision detection fires) and how close the minimum gap gets.

This profile is important for two specific Step 17 questions:

- Does the BSM's hard-braking flag (if implemented) propagate faster than the radar can detect the gap closure?

- Does the fusion KF's lead_a_radar estimate track the emergency deceleration quickly enough to provide useful feedforward, or does it lag so badly it's counterproductive?

The answer to the second question will tell you something important about whether the fusion KF's Q[2,2] — the leader acceleration process noise — is tuned appropriately for extreme events versus nominal following.

Recommended Parameter Values

For a highway CACC validation scenario, 30 m/s (108 km/h) cruise speed is the natural operating point — it's within the range of PATH field tests and matches the headway and gain calibrations in the design notes. Vehicle spacing should give approximately the target headway at cruise speed: with cacc_target_headway_s = 0.6 and s0 = 2.0, desired gap at 30 m/s is 2.0 + 0.6 * 30 = 20 m. Initial positions should reflect this equilibrium so the simulation starts settled rather than spending the first 30 seconds converging from an arbitrary configuration.

A four-vehicle platoon — one lead, three CACC followers — is the minimum useful size for observing string stability. It gives you two inter-vehicle gaps to compare for amplitude growth, which is enough to distinguish stable from unstable behavior. Ten vehicles is more convincing for a research result but four is sufficient for filter validation and faster to debug when something is wrong.

You said: sinusoidal input isn't implemented yet.

sinusoidal input isn't implemented yet. Can I estimate it with values for the lead vehicle, and if so, how do I calculate them and how many to I need?

4:01 PM

Claude responded: Yes, you can approximate it well using the existing ramp transition type, which linearly interpolates between speed targets.

Devised piecewise linear approximation method with waypoint calculations

Devised piecewise linear approximation method with waypoint calculations

Yes, you can approximate it well using the existing ramp transition type, which linearly interpolates between speed targets. The result is a piecewise-linear approximation of the sine — each segment is a straight line in speed, meaning the acceleration profile is piecewise-constant (a staircase) rather than smoothly sinusoidal. For string stability purposes this is entirely adequate because you're measuring amplitude ratios, not waveform fidelity.

The Formula

For a sinusoid with offset v0, amplitude A, and frequency f Hz, the speed at waypoint i is:

T = 1 / f                          (period in seconds)

t_i = warmup_s + i * (T / N)       (time of waypoint i)

v_i = v0 + A * sin(2π * i / N)     (speed at waypoint i)

Where N is the number of waypoints per period, and i runs from 0, 1, 2, ... through however many cycles you want.

How Many Waypoints Per Period

The piecewise-linear approximation error — the maximum deviation from the true sine — is:

max_error = A * (1 - cos(π / N))

| N per period | Spacing at 0.1 Hz | Max speed error | Notes |
| --- | --- | --- | --- |
| 4 | 2.50 s | 0.59 m/s | Triangular wave — too coarse |
| 8 | 1.25 s | 0.15 m/s | Good enough for string stability |
| 12 | 0.83 s | 0.07 m/s | Better, still manageable row count |
| 16 | 0.625 s | 0.04 m/s | Diminishing returns |

N=8 is the practical recommendation. The 0.15 m/s speed error corresponds to roughly 0.12 m/s² acceleration error — about 10% of the sinusoid's acceleration amplitude of A * 2πf = 2.0 * 2π * 0.1 ≈ 1.26 m/s². That's acceptable for measuring amplitude ratios across the platoon.

How Many Cycles

You need at least 3 complete cycles after the warmup for a reliable amplitude measurement, and 5 is better. The warmup period lets transients settle; the cycles after it are the measurement window. At 0.1 Hz each cycle is 10 seconds, so 5 cycles = 50 seconds of oscillation. Total run: 60s warmup + 50s oscillation = 110 seconds.

Worked Example

With v0=30.0, A=2.0, f=0.1 Hz, T=10s, N=8, warmup_s=60:

v_i = 30.0 + 2.0 * sin(2π * i / 8)

= 30.0 + 2.0 * sin(i * π/4)

Warmup rows (2 rows):

| time_s | command | value | transition | note |
| --- | --- | --- | --- | --- |
| 0.00 | set_speed | 30.000 | step | warmup start |
| 60.00 | set_speed | 30.000 | ramp | sine start (i=0) |

Oscillation rows — one period shown, repeat for 5 cycles:

| i | time_s | value (m/s) | transition |
| --- | --- | --- | --- |
| 0 | 60.00 | 30.000 | ramp |
| 1 | 61.25 | 31.414 | ramp |
| 2 | 62.50 | 32.000 | ramp |
| 3 | 63.75 | 31.414 | ramp |
| 4 | 65.00 | 30.000 | ramp |
| 5 | 66.25 | 28.586 | ramp |
| 6 | 67.50 | 28.000 | ramp |
| 7 | 68.75 | 28.586 | ramp |
| 8 | 70.00 | 30.000 | ramp |

Then continue with i=9 through i=40 (five complete periods), incrementing time_s by 1.25 each row and computing 30.0 + 2.0 * sin(i * π/4).

Final row:

| time_s | command | value | transition | note |
| --- | --- | --- | --- | --- |
| 110.00 | set_speed | 30.000 | step | hold at end |

Total rows: 2 warmup + 41 oscillation + 1 final = 44 rows. Easily done in Excel.

Generating the Values

Rather than computing 40 sines by hand, paste this into a Python terminal or a spare sheet in Excel:

python

import math

warmup = 60.0

A = 2.0

v0 = 30.0

f = 0.1

N = 8

cycles = 5

T = 1 / f

for i in range(N * cycles + 1):

t = warmup + i * (T / N)

v = v0 + A * math.sin(2 * math.pi * i / N)

print(f"{t:.2f}\tset_speed\t{v:.4f}\tramp")

Or in Excel, with i in column A starting at 0:

time_s  = 60 + A1 * 1.25

value   = 30 + 2 * SIN(2 * PI() * A1 / 8)

One Practical Note

Because the sine starts at v=v0 (i=0 gives sin(0)=0), the transition from the warmup constant speed into the oscillation is seamless — no discontinuity in speed at t=60s, only a change in the rate of speed change. This avoids an artificial impulse at the start of the measurement window, which is important because that kind of artifact would contaminate the amplitude measurement in the first cycle.