TABLE OF CONTENTS - 1. Algorithm for Blade Geometry and Performance Calculations - 1.1. Step 1: Definition of Analysis Parameters - 1.2. Step 2: Calculate Flow Parameters - 1.2.1. Effective Lambda ($\\lambda\_{E,i}$) Calculation: - 1.3. Step 3: Lift Coefficient and Epsilon Calculation - 1.4. Step 4: Calculation of Coefficients for Each $\\lambda\_0$ - 1.4.1. Local Coefficients Calculation Algorithm - 1.4.2. Blade Coefficients - 1.5. Step 5: RPM and Power Calculation - Rotational Speed (N): - Power calculation based on C_P (P) - Torque (M): - Mechanical Power based on Torque (P_M) - Drag Force (F) calculation

# 1. Algorithm for Blade Geometry and Performance Calculations

## 1.1. Step 1: Definition of Analysis Parameters

The algorithm begins by defining the range of values for λ0. By default the values are between 0 and 60. This parameter represents the tip speed ratio, which is the ratio of the blade tip speed to the wind speed, and it is used to calculate the aerodynamics performance at different rotor conditions.

λ0∈1×10-6,0.2,0.4,…,60.0

Higher values indicate faster blade tip speeds relative to the wind speed, which can affect the flow angle and subsequently the aerodynamic performance of the rotor.

## 1.2. Step 2: Calculate Flow Parameters

The blade is divided into multiple sections along its length, and the flow parameters are calculated for each section. So each segment of the blade is analyzed independently to determine the local flow conditions.

### 1.2.1. Effective Lambda (λE,i) Calculation:

The effective lambda is calculated based on the local tip speed ratio, axial induction factor, and tangential induction factor. The effective lambda is used to determine the flow angle and attack angle at each section of the blade.

Initially, the (λi) for each section is calculated based on the local radius of the blade section.

λiRi,λ0=λ0RiR

where λi is the local tip speed ratio, Ri is the radius at the blade section, and R is the total radius of the blade (on the hub).

Next is (θi) in radians calculated, which is an angle based on the local tip speed ratio.

θiλi=13arctanλi+π3

An Axial Induction Factor (ki) is calculated for each segment based on the flow angle and the local tip speed ratio. The axial induction factor represents the reduction in the axial velocity due to the presence of the rotor.

kiλi,θi=cosθiλi2+1

A Tangential Induction Factor (hi) is calculated for each segment based on the flow angle and the local tip speed ratio. The tangential induction factor represents the reduction in the tangential velocity due to the presence of the rotor.

hiλi,ki=1+1-ki2λi2

This value is dependent on the axial induction factor and the local tip speed ratio and its between 0 and 1.

Using the above quantities it is possible to calculate the effective lambda (λE,i) for each segment of the blade. Essentially the effective λ is a corrected lambda value which takes into account the axial and tangential induction factors.

λE,iλi,ki,hi=λi1+hi1+ki

Using effective λEi for the i-th segment, it is possible to calculate the Flow Angle (ϕi) for that blade segment. The flow angle is the angle between the wind direction and the resultant velocity that the tip of the blade experiences (due to rotation).

ϕi=arctan1λE,i

The Attack Angle Calculation (ai) is then determined based on the flow angle and the twist angle of the blade. The attack angle is the angle between the chord line of the blade and the resultant velocity that the blade experiences.

ai=ϕi-θi+β

where β is the pitch angle of the blade.

This set of calculations provides the flow parameters for each segment of the blade, which are essential for determining the aerodynamic performance of the rotor.

## 1.3. Step 3: Lift Coefficient and Epsilon Calculation

Using the flow parameters calculated in the previous step (and specifically the local attack angle), the lift coefficients (CL) are determined for each segment of the blade.

CL=fairfoil specificai

where f is the airfoil lift coefficient function that relates the attack angle to the lift coefficient. This is specific to the airfoil profile used in the blade design.

Epsilon Calculation (Ε):

For different ranges of attack angles (α), we can define the following fE function to calculate the epsilon value. The epsilon value is used to correct the lift coefficient based on the attack angle.

fEai=3×10-5α2-0.037α-0.218,ai<-57×10-8α4+2×10-6α3+0.0003α2+α+0.0159,-5≤α<292.5×10-8α4-6.0513×10-6α3+0.0001018693α2+0.05811877α-1.2164α≥29

The epsilon value (in radians) is used to correct the lift coefficient based on the attack angle. The lift coefficient is then adjusted based on the epsilon value.

Εai=arctanfEaiCLai

## 1.4. Step 4: Calculation of Coefficients for Each λ0

After CL and Ε are calculated for each segment of the blade, the force and moment coefficients are determined for each λ0 value. This process involves updating/recalculating values. The aim of this step is to calculate the force coefficient (Cf), moment coefficient (Cm), and the final λ0 value for a given λ0 value. These coefficients are essential for determining the performance characteristics of the blade.

An overview of the calculations for each segment of the blade is presented in the following flowchart:

Figure 1: Calculation of Blade coefficients for a specific λ0

The process below starts by assuming a provisional value for the λ0, then given that value, each segment of the blade is examined to determine the lift coefficient, epsilon, and other parameters. The local force and local moment coefficients are then calculated for each segment, and these are integrated over the blade length to determine the overall coefficients for force and moment.

### 1.4.1. Local Coefficients Calculation Algorithm

For the m-th segment of the blade (denoted by m), the Axial Force Coefficient (Am,n) is calculated as follows:

Am,n=CLm,λ0cosΕm,λ0⋅3⋅cm4⋅π⋅rm

where - CLm,λ0 is the lift coefficient for the m-th segment and λ0 value,

- cm is the chord length of the m-th segment

- ri is the radius at the m-th blade segment.

Next the Induction Factor (Gm,λ0) is calculated:

Gm,λ0=Am,λ0⋅cosϕm,λ0-Εm,λ02⋅sinϕm,λ02

where:

- ϕm,λ0 is the flow angle in radians for the m-th segment of λ0, and

- Εm,λ0 is the epsilon value in radians.

Next the Axial Induction Factor (krm,λ0) is recalculated:

krm,λ0=1-Gm,λ01+Gm,λ0

The local Force Coefficient (frm,λ0) is calculated using:

frm,λ0=1-krm,λ02⋅rmR

Using the kr value, an updated value of the Εr is calculated for the m-th segment , λ0:

Εrm,λ0=Am,λ0⋅sinϕm,λ0-Εm,λ0sin2⋅ϕm,λ0

the next step is to update the Tangential Induction Factor (hm,n), using the following formula:

hm,n=1+Εm,λ01-Εm,λ0

The local Moment Coefficient (mrm,λ0) is calculated:

mrm,λ0=1+km,λ02⋅Εm,λ0tanϕm,λ0⋅riR2

Finally, the local tip speed ratio for the m-th specimen (λ0lm,λ0) is calculated. Note that the value ultimately depends on the λ0 value.

λ0lm,λ0=Rrm⋅1+km,λ01+hm,λ0⋅1tanϕm,λ0

### 1.4.2. Blade Coefficients

After calculating the above parameters for all sections, the overall blade coefficients for force (Cf) and moment (Cm) are integrated over the blade length:

Firstly, the blade’s Coefficient of Force (CF) is given by:

CF=2R0Rfrr,λ0 dr

where fr is the force distribution along the blade.

By default the blade is assumed to be divided into segment of equal length with dr being the length of each segment. So, using the trapezoidal rule, the discretized version of the above equation is:

CF=2Ri=1Nfrri,λ0-fr1,λ0+fr(N,λ02⋅dr

The Blade’s Coefficient of Moment (CM) is calculated as:

CMλ0=2R0Rmrr,λ0 dr

where:

- mrm,λ0 is the local moment coefficient on the m-th segment of the blade.

And the Tip Speed Ratio of the blade λ0f, is calculated using the local tip spee ratio λ0l.

λ0,bλ0=1R0Rλ0lr,λ0 dr

## 1.5. Step 5: RPM and Power Calculation

The next step is to perform calculations for different wind speeds to determine the rotational speed, power, torque, and force generated by the rotor at a range or operational conditions. These calculations are based on the coefficients obtained in the previous steps.

### Rotational Speed (N):

The rotational velocity of the rotor is calculated based on the wind speed and the blade geometry. The formula for rotational speed of the blade is given by:

Nλ0,b,U=λ0,b⋅60⋅U2πR

where - λ0,b is the length along the blade and - U is the wind speed in meters per second.

### Power calculation based on C_P (P)

The power generated by the rotor is calculated based on the power coefficient, air density, blade radius, and wind speed. The formula for power is given by:

Pλ0,b,U=12⋅CPλ0,b⋅ρ⋅πR2⋅U3

where: - CP is the power coefficient,

- ρ is the air density,

- R is the blade radius, and

- U is the wind speed.

### Torque (M):

The torque of the blade at a given wind speed is calculated based on the power coefficient, air density, blade radius, and wind speed. The formula for torque is given by:

Mλ0,b,U=CMλ0,b⋅0.6⋅U2⋅πR3

### Mechanical Power based on Torque (PM)

Another way to calculate the power generated by the rotor is based on the torque and rotational speed. The mechanical power is calculated as:

PMλ0,b,U=Nλ0,b,U⋅Mλ0,b,U⋅2π60

### Drag Force (F) calculation

The total drag force generated by the rotor is calculated based on the force coefficient, air density, blade radius, and wind speed. The formula for force is given by:

Fλ0,b,U=CFλ0,b⋅0.6⋅U2⋅πR2

where CF is the drag force coefficient.