- 1. Blade Aerodynamic Properties Calculation

  - 1.1. Overview

  - 1.2. Class Initialization

  - 1.3. Main Calculation of Blade Aerodynamics

    - 1.3.1. Calculation of Section Radii

    - 1.3.2. Calculation of Lambda Effective

    - 1.3.3. Thickness Distribution Factor

    - 1.3.4. Calculation of Angle of Attack

    - 1.3.5. Coefficient of Lift CL

    - 1.3.6. Chord Length Calculation

    - 1.3.7. Thickness Distribution Calculation

    - 1.3.8. Upper and Lower Surface Chord

  - 1.4. Step 2: Calculation of Flow Parameters and Epsilon (step2_calculate Iflow)

    - 1.4.1. Calculation of Meshgrid and Initial Values

    - 1.4.2. Calculation of I Angle of Attack

    - 1.4.3. Calculation of Coefficient of Lift (CL)

    - 1.4.4. Calculation of Epsilon

  - 1.5. Step 3: Recalculation of Eleven Coefficients (step3_eleven_coeffs)

    - 1.5.1. Recalculated Values

  - 1.6. Step 3: Recalculation of Aerodynamic Coefficients

    - 1.6.1. Calculation of Coefficient A

    - 1.6.2. Calculation of Correction Factor G

    - 1.6.3. Calculation of Correction Factor k

    - 1.6.4. Calculation of Local Load Coefficient fr

    - 1.6.5. (Re) Calculation of Induced Angle Epsilon

    - 1.6.6. Calculation of Induction Factor h

    - 1.6.7. Calculation of Moment Coefficient mr

    - 1.6.8. Calculation of Local Tip Speed Ratio l0

    - 1.6.9. Integration to Calculate Coefficients Cf, Cm, and l0final

- 2. Explanation of Variables

# 1. Blade Aerodynamic Properties Calculation

## 1.1. Overview

The Blade class in Python calculates the aerodynamic properties of a blade, focusing on geometry, aerodynamic coefficients, and performance under varying wind conditions.

## 1.2. Class Initialization

- Blade Radius (R): Radius of the blade in meters.

- Angle Constants (i0, A): Constants for angle calculation.

- Thickness Factor (delta_0): Factor for calculating blade thickness.

class Blade:
    R = 0
    i0 = 5
    A  = 5
    delta_0 = 0.25

## 1.3. Main Calculation of Blade Aerodynamics

The main_calculation function performs the primary calculations for the aerodynamics of the blade sections.

### 1.3.1. Calculation of Section Radii

The radii of the blade sections (bl_Ri) are calculated based on the blade radius and the number of sections.

Ri=Rmax⋅insec,  i∈1,nsec+1

where: - i: Index of the blade section (Starting from 1)

- nsec: Number of blade sections

- Rmax: Maximum blade radius

# Derived calculations
self.bl_Ri = blade_R / no_sections * np.arange(1, no_sections + 1) # section radii

### 1.3.2. Calculation of Lambda Effective

Lambda effective (λEi) is calculated to represent the effective tip speed ratio. It’s a crucial part of the aerodynamic calculations.

λi=λ0⋅RiRmaxif λi<λ0nanif λi>λ0

θirad=13⋅arctanλi+π3

ki=cosθi*λ2+1

hi=1+1−ki2λi2

λEi=λi⋅1+hi1+ki

where:

- hi represents the induction factor [no units]

- ki is a correction factor [no units]

- λi is the local tip speed ratio [[no units]]

- λEi is the effective tip speed ratio [no units]

self.m_lambda_i = self._wt_config.lambda0 * self.bl_Ri / self.R  # xslx.Main$B3:B12
    self.m_lambda_i[self.m_lambda_i > self._wt_config.lambda0] = np.nan # TODO: Does this make sense?

    self.m_theta_i = (1/3) * np.arctan(self.m_lambda_i) + (np.pi / 3) # xslx.Main$C3:C12
    self.m_k_i = np.cos(self.m_theta_i) * np.sqrt(self.m_lambda_i**2 + 1)
    self.m_h_i = np.sqrt(1 + (1 - self.m_k_i**2) / self.m_lambda_i**2)
    self.m_lambdaE_i = self.m_lambda_i * (1 + self.m_h_i) / (1 + self.m_k_i)

### 1.3.3. Thickness Distribution Factor

A factor for thickness distribution is calculated using constants m_unkwonn1 and m_unkwonn2.

munknown,i=munknown2⋅1+exp−munknown1⋅RiRmax

m_unkwonn1 = 0.8
m_unkwonn2 = 0.45
self.m_unkwonn = m_unkwonn2 * (1 + np.exp(-m_unkwonn1 * self.bl_Ri / blade_R))

### 1.3.4. Calculation of Angle of Attack

The angle of attack (bl_a_i) is calculated considering the inflow angle, relative angle, and pitch configuration.

aideg=IiFlowdeg−irdeg+config pitchdeg=

aideg=arctan1λEi⋅180π−Aangle⋅RmaxRi+config pitchdeg

where: - ai is the angle of attack [deg]

- IiFlow is the inflow angle [deg], IiFlow=arctan1λEi⋅180π

- ir is the relative angle [deg], ir=Aangle⋅RmaxRi

- Aangle is a constant [deg]

- λEi is the effective tip speed ratio for section i [no units]

- Ri is the radius of section i [m]

- Rmax is the maximum radius of the blade [m]

self.bl_IiFlow = np.arctan(1 / self.m_lambdaE_i) * 180 / np.pi
self.bl_ir = blade_A * np.sqrt(blade_R / self.bl_Ri)
self.bl_a_i = (self.bl_IiFlow - self.bl_ir) + config_pitch

### 1.3.5. Coefficient of Lift CL

The coefficient of lift (bl_cLcZ_i)???? for each section is determined based on the angle of attack.

bl_cLcZ_i=0.4+0.911⋅irif ir<111.3otherwise

self.bl_cLcZ_i = np.ones(no_sections) * 1.3
self.bl_cLcZ_i[self.bl_ir < 11] = 0.4 + (0.9 / 11) * self.bl_ir[self.bl_ir < 11]

### 1.3.6. Chord Length Calculation

The chord length (bl_c_i) is calculated for each blade section.

ci=m_unkwonni⋅Ri⋅1−ki⋅8⋅πcLcZi⋅configP⋅1+ki⋅λEi⋅λEi2+1

self.bl_c_i = (self.m_unkwonn * self.bl_Ri * (1 - self.m_k_i) * 8 * np.pi) / (
    self.bl_cLcZ_i * config_P * (1 + self.m_k_i) * (
        self.m_lambdaE_i * np.sqrt(self.m_lambdaE_i**2 + 1))
    )

### 1.3.7. Thickness Distribution Calculation

The thickness distribution (δ_mmi) and its percentage (δ_Perci) are computed for each section.

δ_mmi=m_unkwonni⋅0.13⋅1000⋅expRi−Rmaxδ0

δ_Perci=0.1⋅δ_mmici

self.bl_delta_mm = self.bl_c_i * 0.13 * 1000 * np.exp((blade_R - self.bl_Ri) / blade_delta_0)
self.bl_delta_Perc = 0.1 * self.bl_delta_mm / self.bl_c_i

### 1.3.8. Upper and Lower Surface Chord

Calculating the chord positions for the upper and lower surfaces of the blade.

#TODO: this actually seems like the positin of the aerodynamic center based on the chord

self.m_cDown = -self.bl_c_i * 0.26
self.m_cTop = self.bl_c_i * 0.74

## 1.4. Step 2: Calculation of Flow Parameters and Epsilon (step2_calculate Iflow)

This function calculates various aerodynamic parameters related to the flow around the blade, including the I angle of attack, Coefficient of Lift (CL), and Epsilon.

### 1.4.1. Calculation of Meshgrid and Initial Values

First, a meshgrid is created using the range of λ0 (l0rng) and the blade section radii (R_i (bl_ri)).

l0msh,Rimsh=meshgridl0rng,blRi

where: - l0msh: Meshgrid for lambda0 values. - Rimsh: Meshgrid for blade section radii. - l0rng: ndarray of the range of λ0 values. - bl_Ri: ndarray of Blade section radii (Ri).

self.l0rng = np.concatenate(([1e-6], np.arange(0.2, 60.1, 0.2)))
l0no = len(self.l0rng)
self.l0msh, self.Rimsh = np.meshgrid(self.l0rng, self.bl_Ri)

### 1.4.2. Calculation of I Angle of Attack

The I angle of attack is calculated using the meshgrid and a function l0func.

valsλ,Ri=arctanRmaxλi⋅Ri⋅180π

#TODO: check validity

Iattackλ,Ri=valsλ,Ri−ai=arctanRmaxλi⋅Ri⋅180π−ai

where: - vals: Calculated values from l0func. - R: Blade radius. - l0: Lambda0 values. - r: Radial positions.

l0no = len(self.l0rng)
l0func = lambda r, l0: 180 / np.pi * np.arctan(self.R / (l0 * r))
self.vals = l0func(self.Rimsh, self.l0msh)
# I angle of attack
self.Iattack = self.vals - np.tile(self.bl_a_i, (l0no, 1)).T

### 1.4.3. Calculation of Coefficient of Lift (CL)

The Coefficient of Lift (CL) is calculated based on the I angle of attack.

CLλ,Ri=cl_approxIattackλ,Ri

where: - CL: Coefficient of Lift. - cl_approx: Function to approximate CL. - Iattack: I angle of attack.

self.CL = np.vectorize(cl_approx)(self.Iattack)

### 1.4.4. Calculation of Epsilon

Epsilon is calculated using the Coefficient of Lift (CL) and the I angle of attack (Iattack).

Epsilonλ,Ri=arctan3⋅10−5⋅Iattack2λ,Ri−0.037⋅Iattackλ,Ri−0.218CLλ,Ri180π

where: - Epsilon: Induced angle (in degrees). - CL: Coefficient of Lift. - Iattack: I angle of attack.

self.Epsilon = (3e-5 * self.Iattack**2 - 0.037 * self.Iattack - 0.218)
# Additional conditional modifications to Epsilon based on the range of Iattack
self.Epsilon = 180 / np.pi * np.arctan(self.Epsilon / self.CL)

## 1.5. Step 3: Recalculation of Eleven Coefficients (step3_eleven_coeffs)

This step involves recalculating various coefficients based on the aerodynamic properties.

### 1.5.1. Recalculated Values

# Recalculating coefficients based on aerodynamic properties
l0no = len(self.l0rng)

# Tile the blade radii for vectorized operations
ri_l0 = np.tile(self.bl_Ri.reshape(-1, 1), (1, l0no))

## 1.6. Step 3: Recalculation of Aerodynamic Coefficients

This step recalculates various coefficients based on the aerodynamic properties of the blade sections.

### 1.6.1. Calculation of Coefficient A

The coefficient A is calculated for each blade section and λ0 value. It is a function of the Coefficient of Lift (CL), chord length (ci), and the epsilon angle (in degrees).

A=CL⋅3⋅ci4π⋅ril0⋅cosϵ⋅π180

where: - A: Coefficient (2D array) (recalc_A) - CL: Coefficient of Lift (vector), - ci: Chord length (vector of length [no_sections]) - ril0: ?? (vector?? of length []) - ϵ: epsilon angle (vector of length [no_sections])

# A: Coefficient (2D array)
# CL: Coefficient of Lift (vector), c_i: Chord length (vector)
# Calculates the A coefficient for each section and lambda0
# A = CL * 3 * c_i / (4 * π * ri_l0 * cos(π * Epsilon / 180))
self.recalc_A = self.CL * 3 * np.tile(self.bl_c_i, (l0no, 1)).T / \
                (4 * np.pi * ri_l0 * np.cos(np.pi * self.Epsilon / 180))

### 1.6.2. Calculation of Correction Factor G

Correction factor G adjusts for the angle between the chord and the relative wind direction.

G=A2⋅cosvals−ϵ⋅π180sinvals⋅π1802

# G: Correction factor (2D array)
# Corrects for the angle between the chord and the relative wind
# G = A * cos(π / 180 * (vals - Epsilon)) / (2 * (sin(π * vals / 180))²)
self.recalc_G = self.recalc_A * np.cos(np.pi / 180 * (self.vals - self.Epsilon)) / \
                (2 * (np.sin(np.pi * self.vals / 180)) ** 2)

### 1.6.3. Calculation of Correction Factor k

Correction factor k is derived from G as follows:

k=1−G1+G

where: - G is the Correction factor (2D array) for the angle between the chord and the relative wind

# k: Correction factor (2D array)
# k = (1 - G) / (1 + G)
self.recalc_k = (1 - self.recalc_G) / (1 + self.recalc_G)

### 1.6.4. Calculation of Local Load Coefficient fr

The local load coefficient fr is calculated based on the correction factor k and the radial position:

fr=1−k2⋅ril0R

where: - k is the Correction factor (2D array) for the angle between the chord and the relative wind - ril0 is the ?? (vector?? of length []) - R is the blade radius in m (scalar)

# fr: Local load coefficient (2D array)
# fr = (1 - k²) * ri_l0 / R
self.recalc_fr = (1 - self.recalc_k ** 2) * ri_l0 / self.R

### 1.6.5. (Re) Calculation of Induced Angle Epsilon

The induced angle ϵ is recalculated based on the recalculated A values, which is essential for understanding the blade’s aerodynamic behavior:

ϵ=A⋅sinvals−ϵπ180sin2⋅vals⋅π180

# Epsilon: Induced angle (2D array)
# Epsilon = A * sin(π * (vals - Epsilon) / 180) / (sin(2 * π * vals / 180))
self.recalc_Epsilon = self.recalc_A * np.sin(np.pi * (self.vals - self.Epsilon) / 180) / \
                      (np.sin(2 * np.pi * self.vals / 180))

### 1.6.6. Calculation of Induction Factor h

The induction factor $ h $ takes into account the induced angle $ $:

h=1+ϵ1−ϵ

# h: Induction factor (2D array)
# h = (1 + Epsilon) / (1 - Epsilon)
self.recalc_h = (1 + self.recalc_Epsilon) / (1 - self.recalc_Epsilon)

### 1.6.7. Calculation of Moment Coefficient mr

The moment coefficient $ mr $ is a crucial factor for torque calculation:

mr=1+k2ϵril0R2tanπvals180

# mr: Moment coefficient (2D array)
# mr = ((1 + k)²) * Epsilon * ((ri_l0 / R)²) / tan(π * vals / 180)
self.recalc_mr = ((1 + self.recalc_k) ** 2) * self.recalc_Epsilon * ((ri_l0 / self.R) ** 2) / \
                 np.tan(np.pi * self.vals / 180)

### 1.6.8. Calculation of Local Tip Speed Ratio l0

The local tip speed ratio $ l0 $ is calculated for each blade section:

l0=Rril01+k1+htanπvals180

# l0: Local tip speed ratio (2D array)
# l0 = ((R / ri_l0) * (1 + k)) / ((1 + h) * tan(π * vals / 180))
self.recalc_l0 = ((self.R / ri_l0) * (1 + self.recalc_k)) / \
                 ((1 + self.recalc_h) * np.tan(np.pi * self.vals / 180))

# Coefficient of Friction (Cf), Moment (Cm), and Lambda0 (l0final) calculations
# These are vectors resulting from integrating the 2D arrays over the blade sections
self.recalc_Cf = np.zeros(l0no)
self.recalc_Cm = np.zeros(l0no)
self.recalc_l0final = np.zeros(l0no)

for cnt in range(l0no):
    # Trapezoidal integration over blade sections
    self.cf_tmp = np.sum(self.recalc_fr[:, cnt]) - np.sum(self.recalc_fr[[0, -1], cnt]) / 2
    self.recalc_Cf[cnt] = 2 * self.bl_Ri[0] / self.R * self.cf_tmp

    self.cm_tmp = np.sum(self.recalc_mr[:, cnt]) - np.sum(self.recalc_mr[[0, -1], cnt]) / 2
    self.recalc_Cm[cnt] = 2 * self.bl_Ri[0] / self.R * self.cm_tmp

    self.l0_tmp = np.sum(self.recalc_l0[:, cnt]) - np.sum(self.recalc_l0[[0, -1], cnt]) / 2
    self.recalc_l0final[cnt] = self.bl_Ri[0] / self.R * self.l0_tmp

graph TD
    CL[CL: Coefficient of Lift] --> A
    ci[c_i: Chord Length] --> A
    ri_l0[ri_l0: Radial Position] --> A
    A[A: Coefficient A] --> G
    A --> Epsilon
    A --> l0
    vals[vals: Value Array] --> G
    vals --> Epsilon
    vals --> l0
    Epsilon[Epsilon: Induced Angle] --> G
    Epsilon --> h
    Epsilon --> mr
    G[G: Correction Factor] --> k
    k[k: Correction Factor] --> fr
    k --> mr
    k --> l0
    fr[fr: Local Load Coefficient] --> Cf
    Cf[Cf: Coefficient of Friction] --> Output
    mr[mr: Moment Coefficient] --> Cm
    Cm[Cm: Coefficient of Moment] --> Output
    l0[l0: Local Tip Speed Ratio] --> l0final
    l0final[l0final: Final Lambda0] --> Output
    R[R: Blade Radius] --> fr
    R --> mr
    R --> l0

### 1.6.9. Integration to Calculate Coefficients Cf, Cm, and l0final

The coefficients of friction (Cf), moment (Cm), and lambda0 (l0final) are derived by integrating the calculated values over the blade sections. The trapezoidal rule is used for this numerical integration.

# 2. Explanation of Variables

- ri_l0: Radii for each section and lambda0 (2D array).

- self.recalc_A: A coefficient for each section and lambda0 (2D array).

- self.recalc_G: Correction factor G (2D array).

- self.recalc_k: Correction factor k (2D array).

- self.recalc_fr: Local load coefficient (2D array).

- self.recalc_Epsilon: Induced angle (2D array).

- self.recalc_h: Induction factor (2D array).

- self.recalc_mr: Moment coefficient (2D array).

- self.recalc_l0: Local tip speed ratio (2D array).

- self.recalc_Cf, self.recalc_Cm, self.recalc_l0final: Resultant vectors for the coefficient of friction, moment, and lambda0.

| Parameter | Description | First Set in Step |
| --- | --- | --- |
| R | Blade radius in meters | __init__ |
| i0 | Initial angle for NACA4415 | Class definition |
| A | Angle factor for NACA4415 | Class definition |
| delta_0 | Factor for calculating the thickness | Class definition |
| _wt_config | Wind turbine configuration | __init__ |
| no_slices | Number of blade sections | main_calculation |
| bl_Ri | Section radii | main_calculation |
| m_lambda_i | Intermediate lambda values | calculate_lambdaE_i |
| m_theta_i | Theta values for lambda calculation | calculate_lambdaE_i |
| m_k_i | k values for lambda calculation | calculate_lambdaE_i |
| m_h_i | h values for lambda calculation | calculate_lambdaE_i |
| m_lambdaE_i | Final lambda values | calculate_lambdaE_i |
| bl_IiFlow | Ii flow values | main_calculation |
| bl_ir | Intermediate ir values | main_calculation |
| bl_a_i | a_i values | main_calculation |
| bl_cLcZ_i | Lift coefficient values | main_calculation |
| bl_c_i | Chord length values | main_calculation |
| bl_delta_mm | Thickness distribution in mm | main_calculation |
| bl_delta_Perc | Thickness distribution in percentage | main_calculation |
| m_cDown | Downwind chord values | main_calculation |
| m_cTop | Topwind chord values | main_calculation |
| l0rng | Range of lambda values | calculate_i_flow |
| l0msh | Lambda mesh grid | calculate_i_flow |
| Rimsh | Section radius mesh grid | calculate_i_flow |
| vals | Flow angle values | calculate_i_flow |
| Iattack | Attack angle values | calculate_i_flow |
| CL | Lift coefficient values | calculate_i_flow |
| Epsilon | Epsilon values | calculate_i_flow |
| recalc_A | Recalculated A values | step3_eleven_coeffs |
| recalc_G | Recalculated G values | step3_eleven_coeffs |
| recalc_k | Recalculated k values | step3_eleven_coeffs |
| recalc_fr | Recalculated fr values | step3_eleven_coeffs |
| recalc_Epsilon | Recalculated Epsilon values | step3_eleven_coeffs |
| recalc_h | Recalculated h values | step3_eleven_coeffs |
| recalc_mr | Recalculated mr values | step3_eleven_coeffs |
| recalc_l0 | Recalculated l0 values | step3_eleven_coeffs |
| recalc_Cf | Recalculated Cf values | step3_eleven_coeffs |
| recalc_Cm | Recalculated Cm values | step3_eleven_coeffs |
| recalc_l0final | Final recalculated l0 values | step3_eleven_coeffs |
| s4_l0 | Lambda0 values for step 4 | step4_coeffs_collection |
| s4_Cm | Cm values for step 4 | step4_coeffs_collection |
| s4_Cp | Cp values for step 4 | step4_coeffs_collection |
| s4_Cf | Cf values for step 4 | step4_coeffs_collection |
| df | DataFrame of collected coefficients | step4_coeffs_collection |
| s5_N | Rotor speed values | step5_rpm_power |
| s5_P | Power values | step5_rpm_power |
| s5_M | Torque values | step6_torque_calcs |
| s5_P_M | Power values through rotor speed and torque | step6_torque_calcs |
| s5_F | Force values | step6_torque_calcs |

| Parameter | Description | Type | Dimensions | First Set in Step |
| --- | --- | --- | --- | --- |
| no_slices | Number of blade sections | Scalar |  | main_calculation |
| bl_Ri | Section radii | 1D Array | (no_sections) | main_calculation |
| bl_IiFlow | Ii flow values | 1D Array | (no_sections) | main_calculation |
| bl_ir | Intermediate ir values | 1D Array | (no_sections) | main_calculation |
| bl_a_i | a_i values | 1D Array | (no_sections) | main_calculation |
| bl_cLcZ_i | Lift coefficient values | 1D Array | (no_sections) | main_calculation |
| bl_c_i | Chord length values | 1D Array | (no_sections) | main_calculation |
| bl_delta_mm | Thickness distribution in mm | 1D Array | (no_sections) | main_calculation |
| bl_delta_Perc | Thickness distribution in percentage | 1D Array | (no_sections) | main_calculation |
| m_cDown | Downwind chord values | 1D Array | (no_sections) | main_calculation |
| m_cTop | Topwind chord values | 1D Array | (no_sections) | main_calculation |
| l0rng | Range of lambda values | 1D Array | (l0no) | calculate_i_flow |
| l0msh | Lambda mesh grid | 2D Array | (no_sections, l0no) | calculate_i_flow |
| Rimsh | Section radius mesh grid | 2D Array | (no_sections, l0no) | calculate_i_flow |
| vals | Flow angle values | 2D Array | (no_sections, l0no) | calculate_i_flow |
| Iattack | Attack angle values | 2D Array | (no_sections, l0no) | calculate_i_flow |
| CL | Lift coefficient values | 2D Array | (no_sections, l0no) | calculate_i_flow |
| Epsilon | Epsilon values | 2D Array | (no_sections, l0no) | calculate_i_flow |
| recalc_A | Recalculated A values | 2D Array | (no_sections, l0no) | step3_eleven_coeffs |
| recalc_G | Recalculated G values | 2D Array | (no_sections, l0no) | step3_eleven_coeffs |
| recalc_k | Recalculated k values | 2D Array | (no_sections, l0no) | step3_eleven_coeffs |
| recalc_fr | Recalculated fr values | 2D Array | (no_sections, l0no) | step3_eleven_coeffs |
| recalc_Epsilon | Recalculated Epsilon values | 2D Array | (no_sections, l0no) | step3_eleven_coeffs |
| recalc_h | Recalculated h values | 2D Array | (no_sections, l0no) | step3_eleven_coeffs |
| recalc_mr | Recalculated mr values | 2D Array | (no_sections, l0no) | step3_eleven_coeffs |
| recalc_l0 | Recalculated l0 values | 2D Array | (no_sections, l0no) | step3_eleven_coeffs |
| recalc_Cf | Recalculated Cf values | 1D Array | (l0no) | step3_eleven_coeffs |
| recalc_Cm | Recalculated Cm values | 1D Array | (l0no) | step3_eleven_coeffs |
| recalc_l0final | Final recalculated l0 values | 1D Array | (l0no) | step3_eleven_coeffs |
| s4_l0 | Lambda0 values for step 4 | 1D Array | (no_l0) | step4_coeffs_collection |
| s4_Cm | Cm values for step 4 | 1D Array | (no_l0) | step4_coeffs_collection |
| s4_Cp | Cp values for step 4 | 1D Array | (no_l0) | step4_coeffs_collection |
| s4_Cf | Cf values for step 4 | 1D Array | (no_l0) | step4_coeffs_collection |
| df | DataFrame of collected coefficients | DataFrame |  | step4_coeffs_collection |
| s5_N | Rotor speed values | 2D Array | (no_l0, len(u)) | step5_rpm_power |
| s5_P | Power values | 2D Array | (no_l0, len(u)) | step5_rpm_power |
| s5_M | Torque values | 2D Array | (no_l0, len(u)) | step6_torque_calcs |
| s5_P_M | Power values through rotor speed and torque | 2D Array | (no_l0, len(u)) | step6_torque_calcs |
| s5_F | Force values | 2D Array | (no_l0, len(u)) | step6_torque_calcs |
| R | Blade radius in meters | Scalar |  | __init__ |
| i0 | Initial angle for NACA4415 | Scalar |  | Class definition |
| A | Angle factor for NACA4415 | Scalar |  | Class definition |
| delta_0 | Factor for calculating the thickness | Scalar |  | Class definition |
| _wt_config | Wind turbine configuration | Config |  | __init__ |
| m_lambda_i | Intermediate lambda values | 1D Array | (no_sections) | calculate_lambdaE_i |
| m_theta_i | Theta values for lambda calculation | 1D Array | (no_sections) | calculate_lambdaE_i |
| m_k_i | k values for lambda calculation | 1D Array | (no_sections) | calculate_lambdaE_i |
| m_h_i | h values for lambda calculation | 1D Array | (no_sections) | calculate_lambdaE_i |
| m_lambdaE_i | Final lambda values | 1D Array | (no_sections) | calculate_lambdaE_i |