Code-“from netmiko import ConnectHandler

import time

devices = [

{"name": "R1", "port": 5000},

{"name": "R2", "port": 5001},

{"name": "R3", "port": 5002},

{"name": "R4", "port": 5015},

]

configs = {

# ================= R1 =================

"R1": [

"hostname R1",

"int f0/0",

"ip address 156.10.0.1 255.255.255.0",

"no shutdown",

"int f1/0",

"ip address 186.10.0.1 255.255.255.0",

"no shutdown",

"int f1/1",

"ip address 146.10.0.1 255.255.255.0",

"no shutdown",

"router bgp 200",

"neighbor 186.10.0.2 remote-as 100",

"neighbor 146.10.0.2 remote-as 400",

"network 156.10.0.0 mask 255.255.255.0"

],

# ================= R2 =================

"R2": [

"hostname R2",

"int f0/0",

"ip address 166.10.0.1 255.255.255.0",

"no shutdown",

"int f1/0",

"ip address 186.10.0.2 255.255.255.0",

"no shutdown",

"int f1/1",

"ip address 196.10.0.1 255.255.255.0",

"no shutdown",

"router bgp 100",

"neighbor 186.10.0.1 remote-as 200",

"neighbor 196.10.0.2 remote-as 300",

"network 166.10.0.0 mask 255.255.255.0"

],

# ================= R3 =================

"R3": [

"hostname R3",

"int f0/0",

"ip address 176.10.0.1 255.255.255.0",

"no shutdown",

"int f1/0",

"ip address 196.10.0.2 255.255.255.0",

"no shutdown",

"int f1/1",

"ip address 136.10.0.2 255.255.255.0",

"no shutdown",

"router bgp 300",

"neighbor 196.10.0.1 remote-as 100",

"neighbor 136.10.0.1 remote-as 400",

"network 176.10.0.0 mask 255.255.255.0"

],

# ================= R4 =================

"R4": [

"hostname R4",

"int f1/0",

"ip address 126.10.0.1 255.255.255.0",

"no shutdown",

"int f1/0",

"ip address 146.10.0.2 255.255.255.0",

"no shutdown",

"int f1/1",

"ip address 136.10.0.1 255.255.255.0",

"no shutdown",

"router bgp 400",

"neighbor 146.10.0.1 remote-as 200",

"neighbor 136.10.0.2 remote-as 300",

"network 126.10.0.0 mask 255.255.255.0",

"route-map PREPEND permit 10",

"set as-path prepend 400 400 400",

"router bgp 400",

"neighbor 136.10.0.2 route-map PREPEND out"

]

}

for dev in devices:

print(f"\n Connecting to {dev['name']}...")

router = {

"device_type": "cisco_ios_telnet",

"host": "127.0.0.1",

"port": dev["port"],

"fast_cli": False,

"global_delay_factor": 2

}

try:

conn = ConnectHandler(**router)

conn.enable()

conn.send_command("terminal length 0")

print(" Sending config...")

print(conn.send_config_set(configs[dev["name"]]))

print(" Saving config...")

print(conn.send_command("write memory"))

conn.disconnect()

print(f" {dev['name']} DONE")

except Exception as e:

print(f" {dev['name']} ERROR: {e}")

time.sleep(1)

print("\n ALL ROUTERS CONFIGURED SUCCESSFULLY!")

”

Steps-“Here are clear, exam-ready documentation steps for your practical:

📘 Practical Documentation

Title: Implementation of BGP using AS_PATH Attribute

🎯 Aim

To implement Border Gateway Protocol (BGP) and manipulate routing using the AS_PATH attribute in GNS3.

🧰 Requirements

- GNS3 Software

- 4 Routers (R1, R2, R3, R4)

- VPCS (optional for testing)

- Cisco IOS images

- Python (optional for automation using Netmiko)

🗺️ Topology Description

- R1 → AS200

- R2 → AS100

- R3 → AS300

- R4 → AS400

Two paths exist between R1 and R3:

- R1 → R2 → R3

- R1 → R4 → R3

⚙️ IP Addressing Table

| Link | Network |
| --- | --- |
| R1–R2 | 186.10.0.0 |
| R2–R3 | 196.10.0.0 |
| R1–R4 | 146.10.0.0 |
| R4–R3 | 136.10.0.0 |
| R1 LAN | 156.10.0.0 |
| R2 LAN | 166.10.0.0 |
| R3 LAN | 176.10.0.0 |
| R4 LAN | 126.10.0.0 |

🪜 Procedure / Steps

Step 1: Create Topology

- Add 4 routers in GNS3

- Connect as per diagram

- Start all devices

Step 2: Configure IP Addresses

- Assign IP addresses to all router interfaces

- Use no shutdown to enable interfaces

Step 3: Configure BGP

On R1:

- Configure AS200

- Add neighbors (R2, R4)

- Advertise network

On R2:

- Configure AS100

- Add neighbors (R1, R3)

On R3:

- Configure AS300

- Add neighbors (R2, R4)

On R4:

- Configure AS400

- Add neighbors (R1, R3)

Step 4: Verify BGP Neighbors

show ip bgp summary

✔ Ensure all neighbors are in Established state

Step 5: Check Routing Table

show ip route

show ip bgp

✔ Observe multiple paths to same destination

Step 6: Apply AS_PATH Attribute

- Create route-map

- Apply AS_PATH prepend on R4

route-map PREPEND permit 10

set as-path prepend 400 400 400

- Apply to neighbor:

router bgp 400

neighbor 136.10.0.2 route-map PREPEND out

Step 7: Verify AS_PATH

show ip bgp

✔ Check longer AS_PATH via R4

Step 8: Testing

- Ping from R1 to R3 network

ping 176.10.0.2

✔ Traffic should follow shortest AS_PATH

📊 Result

- BGP successfully configured

- Multiple paths created

- AS_PATH attribute manipulated

- Routing preference controlled successfully

🧠 Conclusion

The experiment demonstrates that:

- BGP selects routes based on shortest AS_PATH

- AS_PATH can be modified using prepend

- Network traffic flow can be controlled efficiently

⚠️ Precautions

- Ensure correct AS numbers

- Verify neighbor IPs

- Always use no shutdown

- Apply route-map in correct direction (out)

💬 If You Want

I can also give you:

- 📄 Viva questions + answers

- 📊 Output screenshots explanation

- 🧾 Full report in Word/PDF format

Just tell me 👍

”