Alright — here’s your full clean copy-paste configuration (with router-id fixed + everything included) for your exact topology 👇

🔹 🟢 R4 CONFIG (AS 200)

enable

conf t

hostname R4

interface f0/0

ip address 156.10.0.1 255.255.255.0

no shutdown

interface f0/1

ip address 156.20.0.1 255.255.255.0

no shutdown

router bgp 200

bgp router-id 4.4.4.4

neighbor 156.10.0.2 remote-as 100

neighbor 156.20.0.2 remote-as 100

network 1.1.1.0 mask 255.255.255.0

end

🔹 🟢 R5 CONFIG (AS 100)

enable

conf t

hostname R5

interface f0/0

ip address 156.10.0.2 255.255.255.0

no shutdown

interface f0/1

ip address 10.0.0.1 255.255.255.0

no shutdown

router bgp 100

bgp router-id 5.5.5.5

neighbor 156.10.0.1 remote-as 200

neighbor 10.0.0.2 remote-as 100

network 5.5.5.0 mask 255.255.255.0

! iBGP fix

neighbor 10.0.0.2 next-hop-self

end

🔹 🟢 R6 CONFIG (AS 100)

enable

conf t

hostname R6

interface f0/0

ip address 156.20.0.2 255.255.255.0

no shutdown

interface f0/1

ip address 10.0.0.2 255.255.255.0

no shutdown

router bgp 100

bgp router-id 6.6.6.6

neighbor 156.20.0.1 remote-as 200

neighbor 10.0.0.1 remote-as 100

network 6.6.6.0 mask 255.255.255.0

! iBGP fix

neighbor 10.0.0.1 next-hop-self

end

🔥 LOCAL PREFERENCE (Outbound via R5)

👉 Apply on R5

conf t

route-map LOCAL-PREF permit 10

set local-preference 200

router bgp 100

neighbor 156.10.0.1 route-map LOCAL-PREF in

end

🔥 MED (Inbound via R6)

👉 Preferred path = R6 (lower MED)

On R6:

conf t

route-map SET-MED permit 10

set metric 50

router bgp 100

neighbor 156.20.0.1 route-map SET-MED out

end

On R5:

conf t

route-map SET-MED permit 10

set metric 100

router bgp 100

neighbor 156.10.0.1 route-map SET-MED out

end

🔹 ✅ VERIFICATION

show ip bgp summary

show ip bgp

show ip route

🔥 FINAL RESULT

✔ Outbound → R5 → R4 (Local Pref 200)
✔ Inbound → R4 → R6 (MED lower)
✔ No router-id error ✅
✔ iBGP working (next-hop-self applied) ✅

If something still doesn’t come up (like neighbors not Established), tell me — I’ll debug it with you step-by-step.