Code-

from netmiko import ConnectHandler

devices = [

{"name": "R1", "port": 5000},

{"name": "R2", "port": 5001},

{"name": "R3", "port": 5002},

{"name": "R4", "port": 5003},

]

configs = {

"R1": [

"hostname R1",

"int f1/0",

"ip address 196.10.0.1 255.255.255.0",

"bandwidth 128",

"no shutdown",

"int f1/1",

"ip address 196.20.0.1 255.255.255.0",

"bandwidth 128",

"no shutdown",

"int f0/0",

"ip address 156.10.0.1 255.255.255.0",

"no shutdown",

"router eigrp 1",

"network 196.10.0.0",

"network 196.20.0.0",

"network 156.10.0.0",

"no auto-summary"

],

"R2": [

"hostname R2",

"int f1/0",

"ip address 196.10.0.2 255.255.255.0",

"bandwidth 128",

"no shutdown",

"int f1/1",

"ip address 196.30.0.1 255.255.255.0",

"bandwidth 64",

"no shutdown",

"int f0/0",

"ip address 166.10.0.1 255.255.255.0",

"no shutdown",

"router eigrp 1",

"network 196.10.0.0",

"network 196.30.0.0",

"network 166.10.0.0",

"no auto-summary"

],

"R3": [

"hostname R3",

"int f1/0",

"ip address 196.20.0.2 255.255.255.0",

"bandwidth 128",

"no shutdown",

"int f1/1",

"ip address 196.30.0.2 255.255.255.0",

"bandwidth 64",

"no shutdown",

"int se3/0",

"ip address 176.10.0.1 255.255.255.0",

"bandwidth 64",

"no shutdown",

# PBR

"ip access-list standard PBR-PC3",

"permit 186.10.0.0 0.0.0.255",

"exit",

"route-map PBR permit 10",

"match ip address PBR-PC3",

"set ip next-hop 196.20.0.1",

"exit",

"int se3/0",

"ip policy route-map PBR",

"router eigrp 1",

"network 196.20.0.0",

"network 196.30.0.0",

"network 176.10.0.0",

"no auto-summary"

],

"R4": [

"hostname R4",

"int se3/0",

"ip address 176.10.0.2 255.255.255.0",

"bandwidth 64",

"no shutdown",

"int f0/0",

"ip address 186.10.0.1 255.255.255.0",

"no shutdown",

"router eigrp 1",

"network 176.10.0.0",

"network 186.10.0.0",

"no auto-summary"

]

}

for dev in devices:

print(f"\n Connecting to {dev['name']}...")

router = {

"device_type": "cisco_ios_telnet",

"host": "127.0.0.1",

"port": dev["port"],

"username": "",

"password": "",

"secret": "",

"fast_cli": False   # (prevents command skipping)

}

conn = ConnectHandler(**router)

conn.enable()

output = conn.send_config_set(configs[dev["name"]])

print(output)

conn.save_config()

conn.disconnect()

print("\n ALL ROUTERS CONFIGURED SUCCESSFULLY!")

Steps:= ⚙️ Step 5: Define Device Parameters

In script:

- Specify:

  - Device type → cisco_ios_telnet

  - Host → 127.0.0.1

  - Ports → (5000–5003)

🧠 Step 6: Configure Routers via Script

Script performs:

🔹 R1, R2, R4:

- Hostname configuration

- Interface IP assignment

- Bandwidth configuration

- EIGRP routing

🔹 R3:

- Interface configuration

- EIGRP routing

- PBR Configuration:

  - ACL creation

  - Route-map creation

  - Applying policy on interface

🔁 Step 7: Run the Script

In terminal:

python pbr_setup.py

📊 Step 8: Output Verification

After execution:

- Script connects to each router

- Applies configuration

- Displays CLI output

- Saves configuration

🔍 Step 9: Verify in GNS3

Run on routers:

show ip route
show ip policy
show route-map

🧪 Step 10: Test PBR

From R4:

traceroute 156.10.0.2

✅ Expected Result:

Traffic follows path based on PBR:

- Controlled via R3