Perfect — here is your final clean copy-paste configuration for ALL routers ✅
(This is the working version you just fixed)

🔷 ✅ R1 (IP SLA + Default Route + Static Reachability)

conf t

! SLA to R4 loopback

ip sla 1

icmp-echo 4.4.4.4 source-interface FastEthernet1/0

exit

ip sla 2

icmp-echo 4.4.4.4 source-interface FastEthernet1/1

exit

ip sla schedule 1 life forever start-time now

ip sla schedule 2 life forever start-time now

! Tracking

track 1 ip sla 1 reachability

track 2 ip sla 2 reachability

! Default routes with tracking

ip route 0.0.0.0 0.0.0.0 196.10.0.2 track 1

ip route 0.0.0.0 0.0.0.0 196.30.0.2 track 2

! IMPORTANT (reachability to R4 networks)

ip route 196.20.0.0 255.255.255.0 196.10.0.2

ip route 196.40.0.0 255.255.255.0 196.30.0.2

end

wr

🔷 ✅ R2

conf t

! Route to R4 loopback

ip route 4.4.4.4 255.255.255.255 196.20.0.2

! Return path to R1 LAN

ip route 156.10.0.0 255.255.255.0 196.10.0.1

end

wr

🔷 ✅ R3

conf t

! Route to R4 loopback

ip route 4.4.4.4 255.255.255.255 196.40.0.2

! Return path to R1 LAN

ip route 156.10.0.0 255.255.255.0 196.30.0.1

end

wr

🔷 ✅ R4 (Loopback + Proper Return Routing)

conf t

! Loopback (SLA target)

interface loopback0

ip address 4.4.4.4 255.255.255.255

! Return routes (VERY IMPORTANT)

ip route 196.10.0.0 255.255.255.0 196.20.0.1

ip route 196.30.0.0 255.255.255.0 196.40.0.1

end

wr

🔥 FINAL RESULT

✅ R1 uses IP SLA to choose best path
✅ R2/R3 forward traffic normally
✅ R4 sends replies via correct path
✅ Full bidirectional communication
✅ Automatic failover working

🔍 VERIFY (RUN THESE)

On R1:

show track

show ip route

show ip sla statistics

🎯 Expected

Reachability is Up

S* 0.0.0.0/0 via 196.10.0.2

via 196.30.0.2

🧠 One-line summary

R1 decides path using SLA, R4 ensures correct return — making the network stable and failover-ready

If you want, I can give:
✅ Viva answers
✅ Diagram explanation
✅ Failover demo steps

Just tell 👍

Perfect 🔥 — this is the most important part for viva/demo

I’ll give you step-by-step failover demo exactly how to show your sir 👇

🎯 🎬 FAILOVER DEMO (LIVE TEST)

🔷 Step 1: Check current working state

On R1:

show track

✅ You should see:

Track 1 Reachability is Up

Track 2 Reachability is Up

show ip route

✅ You should see:

S* 0.0.0.0/0 via 196.10.0.2

via 196.30.0.2

🔷 Step 2: Check traffic path

traceroute 4.4.4.4

👉 It will go via:

- R2 OR

- R3

💥 Step 3: SIMULATE FAILURE (VERY IMPORTANT)

👉 Go to R2

conf t

interface FastEthernet1/1

shutdown

🔴 What you just did

❌ Broke path:

R1 → R2 → R4

🔷 Step 4: Observe failover (on R1)

Wait 5–10 seconds, then:

show track

✅ Expected:

Track 1 Reachability is Down ❌

Track 2 Reachability is Up ✅

show ip route

✅ Now you’ll see:

S* 0.0.0.0/0 via 196.30.0.2

👉 Only R3 path remains

🔷 Step 5: Verify traffic switched

traceroute 4.4.4.4

👉 Now it will go:

R1 → R3 → R4 ✅

🔷 Step 6: Ping test

ping 4.4.4.4

✅ Should still work:

!!!!!

🔁 Step 7: RESTORE LINK

Go back to R2:

conf t

interface FastEthernet1/1

no shutdown

🔷 Step 8: Recovery check

On R1:

show track

✅ Both UP again:

Track 1 Up

Track 2 Up

show ip route

✅ Both routes back:

S* 0.0.0.0/0 via 196.10.0.2

via 196.30.0.2

🧠 Viva Explanation (say this confidently)

“When the primary path fails, IP SLA detects the failure, tracking goes down, and the default route is removed. Traffic is automatically rerouted through the backup path without manual intervention.”

⚡ One-line summary

Fail link → SLA detects → route removed → traffic switches automatically

🚀 Pro Tip (impress your sir)

Run this continuously:

ping 4.4.4.4 repeat 100

Then shutdown link 👉 you’ll see:

- Few drops ❌

- Then stable again ✅

If you want next:
👉 I can give diagram explanation (very useful for exam drawing)
👉 Or common viva questions + answers 👍