Enabling AI Agents on the oneM2M Service Layer

An open-source Model Context Protocol (MCP) interworking implementation for oneM2M CSEs

Proposal and project overview  ·  August 2026  ·  Contact: rostanaiautomation@gmail.com

# Summary

We are building the first open-source implementation of oneM2M–MCP interworking: a server that lets AI agents (LLM-based assistants and autonomous agents) discover, read, and manage resources on a oneM2M CSE through the Model Context Protocol, with oneM2M access control policies enforced on every agent action. A working prototype already runs on the ACME CSE as a standard user plugin, together with first performance measurements. We would like to align this work with WI-0122 / TR-0081 [1] and contribute our implementation experience back to oneM2M.

# The problem we address

AI agents are becoming a primary way users and systems interact with software. MCP has emerged as the de-facto standard for connecting agents to external systems (500+ public servers; adopted by major AI vendors and now governed under the Linux Foundation). Yet today an AI agent has no standard way to work with a oneM2M system:

No implementation exists. oneM2M has recognised the topic — WI-0122 "Interworking with AI Agents – oneM2M & MCP" and TR-0081 [1] — but as of August 2026 there is no public open-source implementation to learn from, test against, or standardise from.

The open design questions are unanswered. Published oneM2M expert discussions name authentication, authorization, notification handling, and operational dependability as the unsolved problems of agentic access to IoT systems [2]. These need running code to be explored, not only paper designs.

Adjacent ecosystems are moving faster. Home Assistant and commercial IoT platforms already ship MCP servers; LLM-agent research benchmarks exist only for those platforms. Without an implementation, oneM2M risks being invisible in the agentic-AI ecosystem despite being the international IoT standard.

# What we are building

An MCP server for oneM2M CSEs, released as permissively licensed open source (BSD-3, matching the ACME CSE):

Architecture. The server maps MCP tool calls to oneM2M request primitives and dispatches them through the CSE’s own request pipeline, so validation and <accessControlPolicy> enforcement apply to agent traffic exactly as to any AE. It is implemented as a user plugin for the ACME CSE (v2026.05 plugin architecture) — no fork, no core changes.

Agent-facing tools. Resource discovery (filterCriteria), retrieval, AE registration, container and contentInstance management, subscriptions, and a guarded raw-primitive tool — each described so that agents use oneM2M correctly.

Security model. Each agent is mapped to a oneM2M originator with dedicated access control policies; the CSE’s ACP evaluation is the authorization boundary for agent actions — directly addressing the authorization question raised in the WI-0122 context.

Evaluation. A benchmark suite for agent access to a standards-based service layer — to our knowledge the first of its kind — covering discovery, ACP-restricted access, and subscription management.

# Current status (working prototype)

The prototype is functional today on ACME CSE 2026.05.1:

End-to-end verified: an MCP client (as an LLM agent would connect) registers an AE, creates containers, stores and reads data points, and discovers resources — all through oneM2M primitives.

Measured overhead of the MCP layer is approximately 2 ms per operation (p50: 6.2 ms via MCP vs 3.8 ms direct HTTP for contentInstance creation; 4-core commodity hardware).

Sustained throughput through the agent path: 180–240 tool calls per second — ample for agentic workloads.

# Why this is needed now

It turns standards intent into evidence. WI-0122 and TR-0081 are in early drafts; an open reference implementation provides the implementation experience, gap lists, and test material that make the normative work concrete.

It answers oneM2M’s adoption challenge. The learning curve is oneM2M’s most cited adoption barrier. An agent that speaks natural language and uses oneM2M correctly on the user’s behalf is the most direct developer-experience improvement available.

Timing. The machine-readable specification effort, the Release 6 work programme, and the 2026 hackathon season (AIoT theme) all point the same direction; this project is ready to demonstrate at these venues.

# Roadmap and how we would like to engage

Q3 2026 — Public release: hardened security model (dedicated agent originators + ACPs), subscription-to-MCP notification bridging, public GitHub repository and documentation.

Q4 2026 — Evaluation: publish the agent benchmark and results; demonstrate at the oneM2M / ESTIMED hackathons.

Ongoing — Contribution: share implementation findings with the WI-0122 / TR-0081 work; align tool semantics with the emerging specification and adapt as it evolves.

We welcome feedback, and would be glad to present a live demonstration to interested oneM2M members.

# References

[1] oneM2M Release 6 work programme — WI-0122 "Interworking with AI Agents – oneM2M & MCP" and TR-0081 "AI Agent Interworking": https://www.onem2m.org/technical/published-specifications/release-6

[2] I. Friese (Deutsche Telekom), "Early experiments combining Agentic AI, MCP and oneM2M", oneM2M executive viewpoint: https://www.onem2m.org/membership/executive-viewpoints/968-ingo-friese-ai-mcp

[3] oneM2M TP#72 report — Release 6 timeline and AI work items: https://www.onem2m.org/membership/executive-viewpoints/983-tp72-malaysia-iot-standardization

[4] Model Context Protocol specification: https://modelcontextprotocol.io

[5] ACME oneM2M CSE (open-source, plugin architecture): https://github.com/ankraft/ACME-oneM2M-CSE