MCP PCB EMCopilot Framework

Comprehensive PCB Design Review,
EMC Analysis & Signal Integrity Platform

____________________________________________________________

| Version | v0.2.0 (Document Rev 1.0) |
| --- | --- |
| Date | 2026-03-12 |
| Author | Adam Engelbrecht (RFingAdam) |
| License | Apache-2.0 |
| Repository | github.com/RFingAdam/mcp-pcb-emcopilot |

TABLE OF CONTENTS

Right-click and select 'Update Field' to populate Table of Contents

# 1. Framework Overview

## 1.1 What is MCP PCB EMCopilot

MCP PCB EMCopilot is a Model Context Protocol (MCP) server that provides 93 specialized tools for AI-assisted PCB design review, electromagnetic compatibility (EMC) analysis, signal integrity (SI) verification, and power integrity (PI) assessment. It enables AI assistants such as Claude Code and OpenAI Codex CLI to perform deep, physics-based analysis of printed circuit board designs.

The framework parses real PCB layout files from major EDA tools (KiCad, Altium, ODB++, Gerber, IPC-2581) and applies industry-standard formulas and heuristics to identify design issues before they reach the prototype stage. This "shift-left" approach to EMC and SI compliance can save weeks of redesign time and thousands of dollars in failed compliance testing.

## 1.2 Architecture Overview

The system follows a layered architecture with clean separation between protocol handling, data parsing, analysis computation, and output generation:

MCP Protocol Layer: Exposes 93 tools via the Model Context Protocol standard, enabling any MCP-compatible AI client to invoke PCB analysis functions.

Session Manager: Maintains parsed design state across multiple tool calls within a review session.

Parser Layer: Supports 8 input formats including KiCad, ODB++, Gerber, IPC-2581, Altium, STEP, PDF schematics, and BOM files.

Analyzer Layer: 8 analysis domains covering EMC/EMI, signal integrity, power integrity, thermal, DFM, RF/antenna, ESD, and high-speed digital interfaces.

Report & Visualization Layer: Generates SVG board renders, PNG exports, and professional DOCX reports with embedded images and findings.

## 1.3 Key Value Proposition

MCP PCB EMCopilot provides automated, AI-orchestrated design review that catches EMC, signal integrity, and manufacturing issues before the first prototype is built. Key benefits include:

Pre-compliance EMC checking: Predict radiated emissions and compare against FCC/CISPR limits before going to the test lab.

Signal integrity verification: Validate impedance, crosstalk, timing, and eye diagrams for high-speed interfaces (DDR4/5, PCIe, USB, Ethernet).

Power integrity analysis: Check PDN impedance, decoupling strategy, and VRM placement.

Design for manufacturing: Verify solder paste ratios, component placement, assembly constraints, and tolerance stackups.

Automated report generation: Professional DOCX reports with annotated board images, severity-ranked findings, and actionable recommendations.

# 2. Architecture Diagrams

## 2.1 System Architecture

The following diagram shows the end-to-end architecture of the MCP PCB EMCopilot framework, from MCP client interaction through parsing, analysis, and report generation.

Figure 1: MCP PCB EMCopilot System Architecture

## 2.2 PCB Stackup Cross-Section

The framework includes stackup analysis capabilities. Below is a sample 6-layer stackup showing the layer arrangement, copper weights, and dielectric materials that the tool can parse and analyze from design files.

Figure 2: Sample 6-Layer PCB Stackup Cross-Section

## 2.3 EMI Analysis Flow

EMI analysis follows a signal-chain approach, modeling the path from clock/signal sources through PCB traces (acting as transmission lines) to unintentional antenna structures, predicting radiated emissions and comparing against regulatory limits.

Figure 3: EMI Analysis Signal Flow

## 2.4 EMC Test Setup Reference

The compliance prediction tools model standard EMC test configurations. Below is a CISPR 25 radiated emissions test setup, which is the reference geometry used for automotive EMC predictions.

Figure 4: CISPR 25 Radiated Emissions Test Setup

# 3. Tool Categories (93 Tools)

The 93 MCP tools are organized into functional categories. Each tool accepts JSON parameters via the MCP protocol and returns structured JSON results.

## 3.1 Parsers & Session Management (14 tools)

These tools handle loading PCB design files and extracting structured data. The parse_layout tool returns a session_id that subsequent tools use to query the parsed design.

| Tool Name | Description | Key Parameters |
| --- | --- | --- |
| pcb_parse_layout | Parse PCB layout file (KiCad, ODB++, Gerber, IPC-2581, Altium) | file_path |
| pcb_parse_schematic_pdf | Parse PDF schematic for components and net labels | file_path, session_id |
| pcb_parse_step | Parse STEP file for 3D mechanical review | file_path |
| pcb_get_components | Get component list from parsed design | session_id, filter |
| pcb_get_nets | Get net list from parsed design | session_id, filter |
| pcb_get_traces | Get trace summary from parsed design | session_id, net_name, layer |
| pcb_get_vias | Get via list from parsed design | session_id |
| pcb_get_board_outline | Get board outline dimensions and area | session_id |
| pcb_get_stackup | Get layer stackup from parsed design | session_id |
| pcb_get_copper_pours | Get copper pour/zone data per layer | session_id |
| pcb_get_drill_table | Get drill table with sizes and counts | session_id |
| pcb_get_design_rules | Get extracted DRC constraints | session_id |
| pcb_list_sessions | List all active design sessions |  |
| pcb_close_session | Close a design session and free memory | session_id |

## 3.2 Impedance Calculators (7 tools)

Physics-based impedance and transmission line calculators using IPC-2141 formulas (Hammerstad & Jensen for microstrip, Cohn for stripline).

| Tool Name | Description | Key Parameters |
| --- | --- | --- |
| pcb_calc_microstrip_impedance | Surface trace impedance (IPC-2141) | width, height, thickness, Er |
| pcb_calc_stripline_impedance | Buried trace impedance (Cohn) | width, height, thickness, Er |
| pcb_calc_differential_impedance | Differential pair impedance | width, spacing, height, type |
| pcb_calc_cpw_impedance | Coplanar waveguide impedance | width, gap, height, Er |
| pcb_calc_trace_width | Current capacity (IPC-2221) | current, temp_rise, copper_oz |
| pcb_calc_via_stitching | Via stitching density for EMI | frequency, Er |
| pcb_calc_pdn_impedance | PDN impedance frequency sweep | VRM, caps, planes, target_Z |

## 3.3 Signal Integrity (14 tools)

Tools for analyzing signal quality, timing margins, crosstalk coupling, via transitions, and channel loss characteristics.

| Tool Name | Description |
| --- | --- |
| pcb_analyze_timing | Analyze setup/hold margins |
| pcb_analyze_crosstalk | Estimate NEXT and FEXT between traces |
| pcb_analyze_via | Via inductance, capacitance, impedance |
| pcb_analyze_differential_pair | Differential pair routing quality |
| pcb_analyze_length_matching | Trace length matching for signal groups |
| pcb_analyze_mode_conversion | Differential mode conversion / SCD21 |
| pcb_analyze_return_paths | Return path analysis for all HF nets |
| pcb_analyze_return_current | Return current density profile |
| pcb_analyze_return_current_density | Return current density on ref plane |
| pcb_calc_insertion_loss | Frequency-swept S21 and S11 |
| pcb_calc_return_loss | Return loss, VSWR from mismatch |
| pcb_calc_skin_effect | Skin depth and AC resistance |
| pcb_calc_dielectric_loss | Dielectric loss (dB/inch) |
| pcb_calc_eye_diagram | Statistical eye diagram estimation |

## 3.4 EMC / EMI Analysis (14 tools)

Electromagnetic compatibility and interference analysis tools for predicting emissions, analyzing shielding effectiveness, and checking compliance with regulatory standards.

| Tool Name | Description |
| --- | --- |
| pcb_analyze_current_loop | Radiated emissions from current loops |
| pcb_analyze_clock_emi | Clock harmonic EMI envelope (trapezoidal) |
| pcb_analyze_smps_emi | SMPS switching harmonic EMI |
| pcb_analyze_emi_risk | Per-net EMI risk scoring |
| pcb_analyze_shielding | Enclosure shielding effectiveness |
| pcb_analyze_grounding | Grounding topology analysis |
| pcb_analyze_ground_stitch | Ground via stitching optimization |
| pcb_analyze_common_mode | Common-mode noise on diff pairs |
| pcb_analyze_cable_coupling | Cable-to-cable coupling for EMI |
| pcb_analyze_slot_antenna | Ground plane slot as antenna |
| pcb_analyze_trace_antenna | Trace as unintentional antenna |
| pcb_estimate_bandwidth | Signal BW and EMC from rise time |
| pcb_predict_emissions | Radiated emission spectrum vs limits |
| pcb_predict_compliance | EMC compliance prediction |

## 3.5 Power Integrity (5 tools)

Power distribution network analysis including impedance profiling, decoupling capacitor placement, and VRM evaluation.

| Tool Name | Description |
| --- | --- |
| pcb_analyze_pdn | PDN impedance analysis |
| pcb_analyze_decoupling | Decoupling capacitor placement |
| pcb_analyze_vrm | VRM placement and routing analysis |
| pcb_analyze_copper_spreading | Copper area heat spreading |
| pcb_calc_plane_resonance | Power/ground plane resonance |

## 3.6 High-Speed Digital (8 tools)

Interface-specific analyzers for DDR memory, PCIe, USB, and Ethernet with protocol-aware timing and topology validation.

| Tool Name | Description |
| --- | --- |
| pcb_analyze_ddr | DDR memory interface routing analysis |
| pcb_analyze_ddr_timing_budget | Per-lane DDR timing margin (JEDEC) |
| pcb_validate_ddr_topology | DDR topology auto-detect and validate |
| pcb_analyze_usb | USB routing analysis |
| pcb_analyze_ethernet | Ethernet PHY routing analysis |
| pcb_analyze_pcie | PCIe lane routing analysis |
| pcb_validate_pcie_lanes | PCIe lane skew validation |
| pcb_calc_pcie_link_budget | PCIe insertion loss budget |

## 3.7 Thermal Analysis (2 tools)

| Tool Name | Description |
| --- | --- |
| pcb_analyze_thermal | Component thermal dissipation analysis |
| pcb_analyze_thermal_via | Thermal via array analysis |

## 3.8 DFM / Manufacturing (5 tools)

| Tool Name | Description |
| --- | --- |
| pcb_analyze_placement | Component placement manufacturability |
| pcb_analyze_assembly | Board assembly process considerations |
| pcb_analyze_solder_paste | Solder paste stencil design analysis |
| pcb_analyze_tolerance | Manufacturing tolerance stackup |
| pcb_get_manufacturing_notes | Fab notes and material specs |

## 3.9 ESD Protection (1 tool)

| Tool Name | Description |
| --- | --- |
| pcb_analyze_esd | ESD protection circuit analysis |

## 3.10 Classification & Detection (4 tools)

Automated classification of nets, interfaces, and overall design type to drive context-aware analysis selection.

| Tool Name | Description |
| --- | --- |
| pcb_classify_design | Classify design type with complexity score |
| pcb_classify_nets | Classify all nets by function with confidence |
| pcb_detect_interfaces | Detect high-speed interfaces and pin counts |
| pcb_cross_reference_schematic | Cross-reference schematic vs layout |

## 3.11 Visualization & Rendering (5 tools)

| Tool Name | Description |
| --- | --- |
| pcb_render_board | SVG board view with components, traces, vias |
| pcb_render_net | SVG highlighting a specific net |
| pcb_render_stackup | SVG cross-section of layer stackup |
| pcb_annotate_board | SVG with annotation overlays |
| pcb_get_emi_hotspots | Identify high-EMI-risk board regions |

## 3.12 Export & Reporting (6 tools)

| Tool Name | Description |
| --- | --- |
| pcb_export_render_png | Export SVG render to PNG file |
| pcb_export_all_renders | Export all standard renders as PNGs |
| pcb_generate_report | Structured report from review results |
| pcb_generate_docx_report | Professional DOCX with embedded images |
| pcb_get_schematic_page | Get schematic page text and annotations |
| pcb_set_review_context | Set design review context and requirements |

## 3.13 3D & Enclosure (3 tools)

| Tool Name | Description |
| --- | --- |
| pcb_get_3d_clearances | Component-to-component 3D clearances |
| pcb_check_enclosure_fit | PCB assembly fits within enclosure |
| pcb_find_split_crossings | Signals crossing ground plane splits |

## 3.14 Orchestration (2 tools)

High-level tools that coordinate multiple analyzers for comprehensive design review.

| Tool Name | Description |
| --- | --- |
| pcb_run_design_review | Full automated multi-domain design review |
| pcb_optimize_ground_stitching | Optimize ground via stitching pattern |

## 3.15 Reference Data & Utilities (3 tools)

| Tool Name | Description |
| --- | --- |
| pcb_get_stackup_templates | Common PCB stackup configurations |
| pcb_get_material_properties | Dielectric properties for PCB materials |
| pcb_trace_return_path | Trace return current path for a specific net |

# 4. Design Review Workflow

The automated design review process follows a structured pipeline that transforms raw PCB layout files into actionable engineering findings. The AI orchestrator (Claude Code or Codex CLI) drives this workflow by calling the appropriate MCP tools in sequence.

Figure 5: Automated Design Review Workflow

## 4.1 Step 1: Parse Layout

The review begins with pcb_parse_layout, which accepts KiCad (.kicad_pcb), ODB++ (.zip/.tgz), Gerber (RS-274X), IPC-2581 (.xml), or Altium files. The parser extracts components, nets, traces, vias, board outline, stackup, and copper pours into a unified data model. A session_id is returned for subsequent queries.

## 4.2 Step 2: Classify Design

pcb_classify_design analyzes the parsed data to determine the design type (RF, mixed-signal, high-speed digital, power, or simple digital) and assigns a complexity score from 1-10. This classification drives which analyzers are selected in Step 4.

## 4.3 Step 3: Detect Interfaces

pcb_detect_interfaces identifies high-speed interfaces present in the design (DDR, PCIe, USB, Ethernet, LVDS, RF) by analyzing net names, component packages, and pin counts. pcb_classify_nets categorizes every net by function (power, ground, clock, data, analog, etc.).

## 4.4 Step 4: Run Domain-Specific Analyzers

Based on the design classification and detected interfaces, the orchestrator selects the relevant analyzers. For a mixed-signal board with DDR4 and USB, this might include:

EMC: clock EMI, current loop analysis, EMI risk scoring

SI: DDR timing budget, USB differential pair, crosstalk, via analysis

PI: PDN impedance, decoupling, VRM analysis

DFM: placement, solder paste, assembly checks

Thermal: power dissipation, thermal via analysis

## 4.5 Step 5: Generate Visualizations

The visualization layer produces SVG renders of the board layout, individual nets, stackup cross-sections, and annotated boards highlighting findings. These are exported as PNG images for embedding in reports.

## 4.6 Step 6: Export Report

pcb_generate_docx_report creates a professional Word document with embedded board renders, severity-ranked findings organized by domain, and actionable recommendations. The report follows a standard template with executive summary, detailed findings, and appendices.

# 5. Supported File Formats

## 5.1 Input Formats

| Format | Extensions | Data Extracted |
| --- | --- | --- |
| KiCad | .kicad_pcb | Full layout data: components, nets, traces, vias, zones, stackup, board outline |
| ODB++ | .zip / .tgz | Complete manufacturing data: all layers, drill data, netlist, components |
| Gerber | RS-274X | Layer-by-layer artwork: copper, mask, silk, drill (Excellon) |
| IPC-2581 | .xml | Industry standard data exchange format with full design intent |
| Altium | .PcbDoc | Altium Designer binary format (requires olefile) |
| STEP | .step / .stp | 3D mechanical model for clearance and enclosure analysis |
| PDF Schematic | .pdf | Schematic pages with text-layer extraction (requires pymupdf) |
| BOM | .csv / .xlsx | Bill of materials for component validation |

## 5.2 Output Formats

| Format | Usage |
| --- | --- |
| SVG | Board renders, net highlights, stackup cross-sections, annotated boards |
| PNG | Exported from SVG renders via cairosvg for embedding in documents |
| DOCX | Professional design review reports with embedded images and structured findings |
| JSON | Structured analysis results returned via MCP protocol |

# 6. Simulation & Analysis Capabilities

The framework implements physics-based calculations grounded in industry standards and peer-reviewed electromagnetic theory. The following subsections detail the key computational methods.

## 6.1 Impedance Calculations (IPC-2141)

Microstrip impedance uses the Hammerstad & Jensen formulation with thickness correction. Stripline impedance uses the Cohn formula. Both account for effective dielectric constant and conductor thickness effects.

| Trace Type | Formula Basis | Notes |
| --- | --- | --- |
| Microstrip | Hammerstad & Jensen (IPC-2141) | Effective width correction for trace thickness |
| Stripline | Cohn (IPC-2141) | Symmetric or offset stripline between planes |
| Differential Pair | Coupled line theory | Coupling factor from trace spacing / height ratio |
| CPW / GCPW | Conformal mapping | Grounded and ungrounded coplanar waveguide |

## 6.2 Current Capacity (IPC-2221)

Trace width calculations use the IPC-2221 current-temperature rise relationship with separate coefficients for internal and external layers. The formula accounts for copper weight, ambient temperature, and maximum allowable temperature rise.

## 6.3 EMI Prediction

Clock and signal EMI prediction uses trapezoidal waveform Fourier analysis to calculate harmonic content. The envelope of harmonics follows the well-known 0 dB/decade, -20 dB/decade, and -40 dB/decade slopes defined by the fundamental frequency, 1/(pi * tau_r) and 1/(pi * tau_f) corner frequencies. Emissions are calculated using the magnetic dipole antenna model for PCB current loops.

## 6.4 Eye Diagram Estimation

Statistical eye diagram calculation models the channel as a lossy transmission line with frequency-dependent conductor loss (skin effect + Hammerstad roughness correction) and dielectric loss. Eye height and width are estimated from the channel impulse response and compared against standard thresholds for the target data rate.

## 6.5 PDN Impedance

PDN impedance profiling models the VRM output impedance, bulk capacitors, MLCC decoupling capacitors, and plane capacitance as parallel RLC networks. The frequency sweep identifies anti-resonance peaks that exceed the target impedance Z_target = V * ripple% / I_max.

## 6.6 Thermal Analysis

Thermal resistance chain calculation models junction-to-ambient paths through the die, package, PCB copper, thermal vias, and convection. Thermal via arrays are analyzed for effective thermal conductivity enhancement.

## 6.7 Via Modeling

Via electrical models calculate parasitic inductance (from via barrel length and diameter), capacitance (from pad-to-antipad geometry), and characteristic impedance. These are used for signal integrity assessment of layer transitions.

## 6.8 Crosstalk Analysis

Near-end crosstalk (NEXT) and far-end crosstalk (FEXT) coupling coefficients are calculated from trace geometry (width, spacing, dielectric height) using coupled microstrip/stripline theory. The analysis accounts for coupling length, rise time, and termination conditions.

# 7. Integration with Other MCP Servers

MCP PCB EMCopilot is designed to work alongside other specialized MCP servers for a complete PCB engineering workflow. The AI orchestrator can invoke tools across multiple MCP servers in a single design review session.

## mcp-emc-regulations

Capability: FCC Part 15, CISPR 11/22/25/32, IEC 61000-4-x

Provides regulatory emission limits and immunity test levels that EMCopilot's compliance prediction tools compare against. Enables automated pass/fail checking with exact limit values at each frequency.

## mcp-nec2-antenna

Capability: NEC2 engine for dipole, yagi, loop, vertical

Simulates unintentional antenna structures identified by EMCopilot's trace and slot antenna analyzers. Provides radiation pattern and gain data for more accurate emission predictions.

## mcp-openems

Capability: Full-wave EM simulation via openEMS

Validates EMCopilot's approximate calculations with full 3D electromagnetic simulation. Useful for complex structures like via transitions, plane resonances, and connector launches.

## mcp-drawio-engineering

Capability: Engineering diagram generation

Generates PCB stackup diagrams, RF block diagrams, and EMC test setup illustrations. Used to create visual documentation for design review reports.

# 8. Getting Started

## 8.1 Installation

Clone the repository and install with uv:

git clone https://github.com/RFingAdam/mcp-pcb-emcopilot.git
cd mcp-pcb-emcopilot
uv pip install -e .

For full feature support, install optional dependencies:

uv pip install -e '.[all]'

This installs: olefile (Altium), openpyxl (Excel BOM), numpy (S-parameters), networkx (ground analysis), pymupdf (PDF schematics).

## 8.2 Configuration

Add to Claude Code:

claude mcp add pcb-emcopilot -- uv run --directory /path/to/mcp-pcb-emcopilot mcp-pcb-emcopilot

Add to Codex CLI:

codex mcp add pcb-emcopilot -- uv run --directory /path/to/mcp-pcb-emcopilot mcp-pcb-emcopilot

Or add to MCP client config file (JSON):

{
  "command": "uv",
  "args": ["run", "--directory", "/path/to/mcp-pcb-emcopilot", "mcp-pcb-emcopilot"]
}

## 8.3 Example Workflow

A typical design review session with Claude Code:

Step 1: Parse the PCB layout

User: "Parse my KiCad board at /designs/my_board.kicad_pcb and run a full design review."

Step 2: Claude Code calls the tools

# AI orchestrator calls these tools automatically:
pcb_parse_layout(file_path='/designs/my_board.kicad_pcb')
pcb_classify_design(session_id='abc123')
pcb_detect_interfaces(session_id='abc123')
pcb_run_design_review(session_id='abc123')
pcb_render_board(session_id='abc123')
pcb_export_all_renders(session_id='abc123')
pcb_generate_docx_report(session_id='abc123', output_path='review.docx')

Step 3: Review the output

# The AI returns a structured summary of findings:
# - 2 CRITICAL: DDR4 trace impedance mismatch, missing return via
# - 5 WARNING: Crosstalk on CLK net, via stub resonance, ...
# - 8 INFO: Copper pour coverage, stackup recommendations, ...
# Full report saved to review.docx

# 9. API Reference Summary

This section provides a condensed reference for all 93 tools organized alphabetically within each category. For complete parameter documentation, refer to the tool descriptions exposed via the MCP protocol (visible in your MCP client's tool listing).

| Tool Name | Description | Key Parameters |
| --- | --- | --- |
| pcb_parse_layout | Parse PCB layout file | file_path: str |
| pcb_parse_schematic_pdf | Parse PDF schematic | file_path: str, session_id?: str |
| pcb_parse_step | Parse STEP 3D file | file_path: str |
| pcb_get_board_outline | Board dimensions | session_id: str |
| pcb_get_components | Component list | session_id: str, filter?: str |
| pcb_get_copper_pours | Copper pour data | session_id: str |
| pcb_get_design_rules | DRC constraints | session_id: str |
| pcb_get_drill_table | Drill sizes/counts | session_id: str |
| pcb_get_manufacturing_notes | Fab notes | session_id: str |
| pcb_get_nets | Net list | session_id: str, filter?: str |
| pcb_get_stackup | Layer stackup | session_id: str |
| pcb_get_traces | Trace summary | session_id: str, net_name?: str |
| pcb_get_vias | Via list | session_id: str |
| pcb_list_sessions | Active sessions |  |
| pcb_close_session | Close session | session_id: str |
| pcb_calc_cpw_impedance | CPW impedance | width, gap, height, Er, thickness |
| pcb_calc_differential_impedance | Diff pair impedance | width, spacing, height, thickness, Er, type |
| pcb_calc_microstrip_impedance | Microstrip impedance | width, height, thickness, Er |
| pcb_calc_stripline_impedance | Stripline impedance | width, height, thickness, Er |
| pcb_calc_trace_width | Trace current capacity | current, temp_rise, copper_oz, layer |
| pcb_calc_via_stitching | Via stitch spacing | frequency, Er |
| pcb_calc_pdn_impedance | PDN impedance sweep | VRM, caps, planes, freq range |
| pcb_analyze_crosstalk | NEXT/FEXT coupling | width, spacing, height, length, rise_time |
| pcb_analyze_differential_pair | Diff pair quality | session_id, net_p, net_n |
| pcb_analyze_length_matching | Length matching | session_id, net_names, tolerance |
| pcb_analyze_mode_conversion | Mode conversion | Z_diff, Z_comm, length, freq |
| pcb_analyze_return_current | Return current profile | width, height, frequency |
| pcb_analyze_return_current_density | Return density map | session_id, net_name |
| pcb_analyze_return_paths | Return path analysis | session_id |
| pcb_analyze_timing | Timing margins | freq, length, setup, hold, rise_time |

| Tool Name | Description | Key Parameters |
| --- | --- | --- |
| pcb_analyze_via | Via characteristics | drill, pad, antipad, length |
| pcb_calc_dielectric_loss | Dielectric loss | freq, Er, Df, length |
| pcb_calc_eye_diagram | Eye diagram | data_rate, length, Er, Df, width, height |
| pcb_calc_insertion_loss | Insertion loss S21 | freq, length, width, height, Er, Df |
| pcb_calc_return_loss | Return loss S11 | Z_line, Z_load |
| pcb_calc_skin_effect | Skin depth/loss | freq, conductivity, roughness |
| pcb_analyze_cable_coupling | Cable coupling | length, spacing, freq, type |
| pcb_analyze_clock_emi | Clock EMI harmonics | freq, amplitude, rise_time, loop_area |
| pcb_analyze_common_mode | Common-mode noise | Z_diff, skew, freq |
| pcb_analyze_current_loop | Loop radiation | freq, current, area |
| pcb_analyze_emi_risk | EMI risk scoring | session_id |
| pcb_analyze_grounding | Grounding topology | session_id |
| pcb_analyze_ground_stitch | Via stitch spacing | frequency, lambda_fraction |
| pcb_analyze_shielding | Shielding SE | material, thickness, freq, aperture |
| pcb_analyze_slot_antenna | Slot as antenna | length, width, freq |
| pcb_analyze_smps_emi | SMPS EMI | freq, duty, current, loop_area |
| pcb_analyze_trace_antenna | Trace as antenna | length, freq |
| pcb_estimate_bandwidth | BW from rise time | rise_time |
| pcb_predict_compliance | Compliance prediction | session_id, standard |
| pcb_predict_emissions | Emission spectrum | session_id, limit_standard |
| pcb_analyze_copper_spreading | Cu heat spreading | area, thickness, power |
| pcb_analyze_decoupling | Decap placement | session_id, ic_name |
| pcb_analyze_pdn | PDN impedance | session_id, rail_name |
| pcb_analyze_vrm | VRM analysis | session_id |
| pcb_calc_plane_resonance | Plane resonance | length, width, Er, height |
| pcb_analyze_ddr | DDR interface | session_id, ddr_type |
| pcb_analyze_ddr_timing_budget | DDR timing budget | session_id, ddr_type, data_rate |
| pcb_analyze_ethernet | Ethernet routing | session_id |
| pcb_analyze_pcie | PCIe lanes | session_id, gen |
| pcb_analyze_usb | USB routing | session_id, usb_version |

| Tool Name | Description | Key Parameters |
| --- | --- | --- |
| pcb_calc_pcie_link_budget | PCIe link budget | gen, trace_len, via_count |
| pcb_validate_ddr_topology | DDR topology | session_id |
| pcb_validate_pcie_lanes | PCIe lane skew | session_id, gen |
| pcb_analyze_thermal | Thermal dissipation | power, theta_ja, T_ambient |
| pcb_analyze_thermal_via | Thermal via array | via_count, drill, length |
| pcb_analyze_assembly | Assembly check | session_id |
| pcb_analyze_placement | Placement check | session_id |
| pcb_analyze_solder_paste | Solder paste | session_id |
| pcb_analyze_tolerance | Tolerance stackup | dimensions, tolerances |
| pcb_analyze_esd | ESD protection | session_id, port_name |
| pcb_classify_design | Design classification | session_id |
| pcb_classify_nets | Net classification | session_id |
| pcb_cross_reference_schematic | Schem vs layout | session_id |
| pcb_detect_interfaces | Interface detection | session_id |
| pcb_annotate_board | Annotated board SVG | session_id, annotations |
| pcb_get_emi_hotspots | EMI hotspot regions | session_id |
| pcb_render_board | Board SVG render | session_id, layers, highlights |
| pcb_render_net | Net SVG render | session_id, net_name |
| pcb_render_stackup | Stackup SVG render | session_id |
| pcb_export_all_renders | Export all PNGs | session_id, output_dir |
| pcb_export_render_png | Export SVG to PNG | svg_content, output_path |
| pcb_generate_docx_report | DOCX report | session_id, output_path |
| pcb_generate_report | Structured report | session_id, format |
| pcb_get_schematic_page | Schematic page data | session_id, page_number |
| pcb_set_review_context | Set review context | session_id, requirements |
| pcb_check_enclosure_fit | Enclosure fit check | session_id, enclosure dims |
| pcb_find_split_crossings | Split plane crossings | session_id |
| pcb_get_3d_clearances | 3D clearances | session_id |
| pcb_optimize_ground_stitching | Optimize stitching | session_id, frequency |
| pcb_run_design_review | Full design review | session_id |

| Tool Name | Description | Key Parameters |
| --- | --- | --- |
| pcb_get_material_properties | Material database |  |
| pcb_get_stackup_templates | Stackup templates |  |
| pcb_trace_return_path | Trace return path | session_id, net_name |

# 10. Common Design Targets

The following tables provide quick-reference impedance targets and material properties commonly used with the framework's calculators.

## 10.1 Interface Impedance Targets

| Interface | Single-Ended (Ohm) | Differential (Ohm) |
| --- | --- | --- |
| General purpose | 50 | - |
| USB 2.0 | - | 90 |
| USB 3.x | - | 85 |
| HDMI | - | 100 |
| DDR4 | 40 | 80 |
| DDR5 | 40 | 80 |
| PCIe Gen3/4 | - | 85 |
| PCIe Gen5/6 | - | 85 |
| Ethernet 100BASE-TX | - | 100 |
| Ethernet 1000BASE-T | - | 100 |
| LVDS | - | 100 |
| SATA | - | 85 |

## 10.2 PCB Material Properties

| Material | Dk (at 1 GHz) | Df (Loss Tan.) | Tg (C) |
| --- | --- | --- | --- |
| FR-4 Standard | 4.2-4.5 | 0.020 | 140-170 |
| FR-4 High-Tg | 4.2-4.4 | 0.018 | 170-210 |
| Rogers RO4003C | 3.38 | 0.0027 | 280 |
| Rogers RO4350B | 3.48 | 0.0037 | 280 |
| Isola I-Speed | 3.63 | 0.0082 | 200 |
| Megtron 6 (R-5775) | 3.4 | 0.002 | 230 |
| Polyimide (Flex) | 3.3-3.5 | 0.003 | 260+ |