# Context of Use Definition

## HPT Blade CHT - Take-off Transient

Program: Advanced Turbine Engine Program
Date: March 2026

### Scope

A conjugate heat transfer (CHT) computational model predicts peak metal temperature on a high-pressure turbine (HPT) blade during take-off transient conditions. The prediction is used for preliminary screening of blade designs against the 1150K temperature limit.

### Question of Interest

Does the peak metal temperature exceed 1150K during take-off? The model must predict peak surface temperature within +/-50K for preliminary screening.

### Model Risk Level

Model influence: MEDIUM (testing confirms key results). Decision consequence: HIGH (blade failure risk). Model Risk Level: MRL 3 per NASA-STD-7009B.

### Operating Envelope

Inlet T: 1600-1800K, Inlet P: 25-35 bar, RPM: 12000-15000, Cooling: 3-5% mainstream, Transient: 30 sec idle to take-off.

### Exclusions

Does not cover: cruise/descent, off-design, deteriorated blades, FOD, vibration/flutter.