# Context of Use Definition

HPT Blade CHT — Cruise Steady-State (COU2)

Program: Advanced Turbine Engine Program

Date: April 2026

Related COU: See aero-evidence-cou1 (Take-off Transient) for the model's primary validated operating point.

# Scope

A conjugate heat transfer (CHT) computational model predicts quasi-steady metal temperature distribution on a high-pressure turbine (HPT) blade at cruise conditions. This COU extends the existing take-off transient model (COU1, MRL 3) to long-duration cruise operation for blade creep-life margin assessment over the mission profile.

The SAME CFD model (same solver configuration, same mesh, same turbulence model) is being re-purposed for a different operating point. No changes to model form or numerics relative to COU1.

# Question of Interest

What is the peak metal temperature at cruise steady-state, and does the creep-damage accumulation rate integrated over the mission cruise segment remain within the life-limiting allowable for the 25,000-hour design target?

Acceptance criterion: peak metal temperature at cruise <= 1080K with 95 percent confidence; integrated creep damage over 25,000 cruise hours <= 0.7 (Miner fraction on creep).

# Model Risk Level

Model influence: HIGH (primary quantitative input to blade life certification margin calculation). Decision consequence: HIGH (blade service life determination; retirement-for-cause interval). Per NASA-STD-7009B risk matrix: HIGH x HIGH = MRL 4.

Note: this is a higher MRL than COU1 (MRL 3) because cruise predictions drive certification-facing life analysis rather than preliminary screening.

# Operating Envelope

Cruise steady-state at FL350, M 0.82:

• Inlet Total Temperature: 1440 to 1460 K (nominal 1450 K)

• Inlet Total Pressure: 17.5 to 18.5 bar (nominal 18.0 bar)

• Rotational Speed: 10,200 to 10,800 RPM (nominal 10,500 RPM)

• Cooling Flow Ratio: 3.3 to 3.7 percent mainstream (nominal 3.5 percent)

• Duration per cycle: continuous during cruise segment (~90 percent of total flight time for long-haul missions)

# Exclusions

Does not cover: take-off (see COU1), climb, top-of-descent throttle transients, approach, hold patterns at unusual altitudes, off-design conditions, deteriorated engines, FOD events.

# Validation Strategy

At the time of this assessment, no cruise-specific experimental validation data exists for this blade geometry. The validation evidence package is re-used from COU1 (cascade rig at take-off Reynolds and Mach numbers). This re-use is the central credibility concern addressed in the credibility assessment narrative §2-§4.