Laptop Purchasing Guide for ANUGA Hydrodynamic Simulations

Based on benchmarking experience with the Lenovo Legion Pro 5 (RTX 5070 8GB / Ryzen 9 9955HX)

# Reference Platform

Our benchmarking baseline is a Lenovo Legion Pro 5 with an RTX 5070 (8GB GDDR7) and Ryzen 9 9955HX CPU running Ubuntu. Key empirical findings:

- VRAM footprint: ~530 bytes/triangle (float64 representation)

- 8GB VRAM supports approximately 15 million triangles — sufficient for a large fraction of real ANUGA use cases

- RTX 5070 laptop GPU performs comparably to a V100 data-centre card at ~200k triangle domain sizes — the RTX 5070 laptop delivers ~670 GB/s memory bandwidth versus ~900 GB/s for the V100 SXM2, putting them in the same performance tier (versus ~2,000 GB/s for an A100)

- The Legion's cooling handles sustained ANUGA GPU runs without significant thermal throttling

# GPU: The Most Critical Component

ANUGA's time-stepper is memory-bandwidth-bound. GPU choice dominates performance far more than CPU or storage. In our benchmarks the GPU path ran roughly 15–17x faster than a single CPU core and about 2.5–3x faster than 16 CPU cores, with near-flat weak scaling as the domain grows.

## VRAM Capacity — Projected Triangle Ceilings

| VRAM | Triangle ceiling | Notes |
| --- | --- | --- |
| 8 GB | ~15M (measured) | Our Legion baseline; covers most operational domains |
| 16 GB | ~30M (projected) | Doubles headroom; recommended for regional high-res runs |
| 24 GB | ~45M (projected) | Approaches workstation territory |
| 16–32 GB (V100) | ~75M (projected) | HPC data-centre card — ~900 GB/s bandwidth (~35% more than RTX 5070 laptop); NCI Gadi V100 nodes |
| 40–80 GB (A100) | ~150M (projected) | ~3x the bandwidth of RTX 5070 laptop (~2x V100); large-scale tsunami/regional flood modelling |

## Recommended GPU Tiers

- RTX 5070 (8GB) — proven baseline; fine for moderate to large domains up to ~15M triangles

- RTX 5070 Ti (16GB) — most cost-effective upgrade; doubles the VRAM ceiling

- RTX 5080 (16GB) — higher bandwidth than 5070 Ti with the same VRAM ceiling

- RTX 5090 (24GB) — ~45M triangle ceiling; worthwhile only for very high-resolution regional domains

Important caveats:

- Avoid AMD consumer GPUs (RDNA) — poor compiler support for the OpenMP target offloading path ANUGA uses

- Check TGP (Total Graphics Power): RTX 5070 laptop variants range 100–150W; higher TGP = better sustained performance

- MUX switch is important — allows direct GPU access bypassing the iGPU, reducing CPU overhead

# CPU: Secondary but Still Matters

ANUGA's CPU parallel path benefits more from clock speed than core count beyond ~8 cores, due to memory bandwidth saturation.

- Ryzen 9 9955HX is excellent overall. On our benchmark machine it presents as a single NUMA node, and OpenMP scales about 4–5x on 16 cores (not linear). The remaining OpenMP-vs-MPI gap comes from false sharing in the solver's flux arrays and serial operators — not from CPU memory topology

- Prefer MPI parallelism over OpenMP threading for CPU runs — separate per-rank address spaces avoid the false-sharing penalty that caps OpenMP scaling (MPI-16 reached ~8x vs a single core, OpenMP-16 ~4–5x)

- For MPI-parallel ANUGA runs, CPU topology matters even less — domain decomposition gives each rank its own address space, sidestepping the false-sharing limit that caps OpenMP

# RAM

- 32 GB minimum, 64 GB strongly preferred

- ANUGA loads large mesh arrays and SWW output files; 16 GB will cause paging on serious domains

- Verify dual-channel configuration — some thin-and-light laptops use soldered single-channel memory

# Storage

- NVMe SSD, 1 TB or larger

- SWW output files from large domains are very large; write speed matters for yieldstep output

- PCIe 4.0 or 5.0 preferred; do not accept PCIe 3.0 in a new 2025/2026 purchase

# Thermal and Power Considerations

ANUGA GPU workloads are sustained, not bursty. Thermal throttling will significantly reduce real-world performance versus spec-sheet claims.

- Favour enthusiast gaming chassis with large cooling systems over thin-and-light designs

- Look for laptops that maintain boost clocks under sustained compute load (check review benchmarks, not just peak specs)

- A MUX switch (direct GPU mode) is important for compute workloads

# Budget Guide (AUD, approx.)

| Budget | Configuration | Capability |
| --- | --- | --- |
| ~$2,500–3,000 | RTX 5070 (8GB), 32 GB RAM | ~15M triangles — matches our Legion baseline |
| ~$3,000–3,800 | RTX 5070 Ti (16GB), 32–64 GB RAM | ~30M triangles — most cost-effective upgrade |
| ~$4,000–5,000 | RTX 5080 (16GB), 64 GB RAM | ~30M triangles + higher bandwidth |
| ~$5,500+ | RTX 5090 (24GB), 64 GB RAM | ~45M triangles — near-workstation capability |

# Recommended Laptop Models

Beyond the Lenovo Legion Pro 5 (our reference), the following models offer equivalent or better thermal and performance characteristics for sustained ANUGA workloads:

## ASUS ROG Strix / Scar series

- Strong thermal performance with large vapour-chamber cooling

- MUX switch standard across the range

- Available with RTX 5070 Ti through 5090; 16–32 GB VRAM options

- ROG Scar 16/18 are the flagship thermal options

## MSI Titan GT / Raider GE series

- MSI Titan GT77 / GT series historically among the best-cooled gaming laptops available

- Raider GE series offers a slightly thinner chassis with good thermals

- Available with RTX 5080/5090 and up to 64 GB RAM

## ASUS ProArt Studiobook 16

- Workstation-class build aimed at creators and researchers

- Available with RTX 5090 and 64 GB RAM

- Better suited than gaming laptops for professional/research contexts if that matters

- Thermals are solid; slightly thicker chassis than Strix

## Dell Alienware m16 / m18

- Excellent sustained performance; Cherry MX keyboard option on m18

- Large chassis (m18) allows aggressive cooling for sustained workloads

- Available with RTX 5080/5090

- Australian retail availability via Dell direct and JB Hi-Fi

## Razer Blade 16 / 18

- Premium build quality; thinner chassis than most competitors

- Available with RTX 5080/5090

- Thermals are adequate but not class-leading — review sustained performance benchmarks before buying

- Higher price premium for the build quality

# What to Avoid

- Thin-and-light / ultrabooks — thermal budget too low for sustained ANUGA runs

- Laptops with <12 GB VRAM — unless empirically validated for your domain sizes, as the RTX 5070 8GB has been for ANUGA (the 5000-series makes 8 GB a budget-tier choice now)

- RTX 4000-series at full price — the GDDR7 bandwidth improvement in the 5000-series is significant

- "AI laptop" marketing with NPU emphasis — check whether GPU specs were compromised to hit a price point

- AMD consumer GPUs (RDNA) — no viable OpenMP offload path for ANUGA

- Single-channel RAM configurations

# When Laptop GPU Stops Making Sense

For domains exceeding ~15–30M triangles, NCI Gadi's V100 and A100 nodes become the right tool. The laptop GPU path excels for:

- Development and debugging — fast iteration without queuing for HPC allocation

- Parameter sweeps on moderate domains

- Offline or field work where HPC access is unavailable

- GPU code development and profiling (Nsight Systems/Compute)

For production runs on large high-resolution domains, Gadi's V100 (16/32 GB) and A100 (40/80 GB) nodes provide VRAM headroom and multi-GPU MPI capability that no laptop can match — and the A100's ~2,000 GB/s memory bandwidth offers a genuine step up beyond what the RTX 5070 can deliver.

Document prepared June 2026. GPU specifications and pricing are evolving rapidly; verify current availability before purchasing.